Number Nine "HawkEye" Windows Application

for the #9GXi board

����������������ͻ

�                �

�  Introduction  �

�                �

����������������ͼ

This document represents any changes to the standard documentation

that occured after printing.  Also any known problems are described

in the troubleshooting section.

����������������ͻ

�                �

�  Place Windows �

�     Option     �

�                �

����������������ͼ

The Place Windows option has been added for HawkEye release 1.31.

This option allows HawkEye to move each window when each window is

created.  With this option enabled, dialog boxes will always appear

within the viewable area.

In Windows, applications can have either sizable windows (e.g.

MS-Paint, Program Manager) or non-sizable windows (e.g. Control Panel

Mouse, various Dialog Boxes.).   When the place windows option is

enabled, sizable windows are re-sized to the size of the viewable

area.  Since sizable window can be initially maximize (e.g. Corel

Draw), the initially maximized window are made non-maximized.

When the place windows option is enabled, non-sizable windows are

moved to the center of the viewable area.  If the non-sizable window

is larger than the viewable area (This can happen frequently when

the user is zoomed in.), then the center of the dialog box appears in

the center of the screen.

�������������������ͻ

�                   �

�  Troubleshooting  �

�                   �

�������������������ͼ

Problem:   A horizontal line appears in the middle of the display when

�������    you are in Virtual mode.

Solution:  Disable memory packing.

The GXi program can be used to disable memory packing, with

the "Use Advanced Utilities / Change TIGA Parameters" menu.

Set "Enable Memory Packing Mode" to 'NO'.

Memory packing is redundant when you are in Virtual mode.

Memory packing is only useful if the display width is NOT

a power of '2' (e.g., 1024).  Virtual modes can be either

1024 or 2048 pixels wide.

Problem:   You want to change a zooming option, but the Hawkeye icon

�������    is not visible.

Solution:  Double click on the wallpaper.  This generates a 'Task List'

of active applications.  Choose 'Hawkeye' from the list.

Problem:   The cursor is hard to find after zooming. You found it after

�������    an extensive search, but not where you expected it to be.

Solution:  Change the 'Zoom Cursor Location' selection.

(See the 'Options Dialog Box' section, above.)

Problem:   The screen jumps when the cursor is moved out of the viewable

�������    area.

Solution:  Select 'Smooth' for 'Cursor Scrolling'.

(See the 'Options Dialog Box' section, above.)

Problem:   The caret is difficult to see when it is at the right corner

�������    of the viewable area.

Solution:  Select 'Center' for 'Caret Scrolling'.

(See the 'Options Dialog Box' section, above.)

Problem:   You need to use [F11] and / or [F12] for another purpose in a

�������    Windows application.

Solution 1:  Select 'Disable' for the Zoom-In key, and / or Hot-Key.

(See the 'Options Dialog Box' section, above.)

Solution 2:  Select 'Custom' for the Zoom-In key.  Make a custom key

selection.

Select 'Custom' for the Hot-Key.  Select a custom key.

(See the 'Options Dialog Box' section, above.)

Problem:   The caret can not always be correctly tracked when it appears

�������    in a child window.  (This can occur in Microsoft Paint.)

Solution:  Remove the Tools and Linesize icons from the drawing area.

Problem:   When a system modal dialog box is active, zooming is enabled,

�������    but the Hawkeye icon is not correctly updated (i.e., the icon

does not display current magnification).

Problem:   When the system menu of a dialog box is active, the 'Zoom-In'

�������    key sometimes generates two key strokes each time it is pressed.

Problem:   Tetris has mis-sized child windows

�������

Solution:  Disable "Place Windows" option in HawkEye

Problem:   Zoomed Cursor is missing when using 8514 compatable

�������    monitor in 1024 by 768 mode.

Solution:  HawkEye Zoom is not currently supported in interlace

mode.  Use non-interlace 8514 resolution (640 by 480)

or upgrade monitor to non-interlace monitor.

Problem:   Norton Disk Doctor Text is distorted when "Place Windows"

�������    option is enable.

Solution:  If a dialog box is placed on top of the Disk Doctor text,

the text will be restored.

Problem:   Using Place Windows with Microsoft Project will cause a

�������	   UAE when list box is pulled down.

Solution:  Trun Place Windows off before running MS Project.  This

will be fixed in the next release.

Problem:   When using Corel Show in 16 BPP mode slides will not be

�������	   seen in the slide sorter screen.

Solution:  Turn the windows optimizations on.  To due this exit

Windows and type GXI at the DOS prompt.  From the first

menu select Advance and then Select #9gxi Boaard

Parameters. Move the highligh to Enable Windows

Optimizations Buffer and press ENTER.  Change the pop up

to YES and press ESC several times to exit the program.

When the message box comes up asking to reporgram the

board select Program.  Restart Windows.  NOTE.. Do not

do this procedure from a DOS shell.