Windows 3.0 Driver Information

for the Number Nine GXi board

����������������ͻ

�                �

�  Introduction  �

�                �

����������������ͼ

Every Windows workstation must have a Windows driver that matches

the workstation's display adapter.  Hardware manufacturers using the

industry-standard Texas Instruments Graphic Architecture (TIGA)

create drivers to take advantage of TIGA's special capabilities.

Number Nine provides the #9GXi Windows 3.0 TIGA driver to optimize

Windows performance with the #9GXi.

This file shows you, step-by-step, how to install the Windows 3.0

TIGA driver on the #9GXi.

������������������������ͻ

�                        �

�  TIGA Display Drivers  �

�                        �

������������������������ͼ

There are three Windows 3.0 TIGA drivers on the #9GXi Windows Setup

disk.  The driver source code for each driver is identical, but the

driver resources (such as the cursors, icons, etc.) vary, based on

display resolution and the number of displayed colors.

Filename      Used for

��������      ��������

TIGA48.DRV    1280 and 1024 resolutions

(4 and 8 bits per pixel;  16 and 256 display colors)

TIGA48LO.DRV  640 and 800 resolutions

(4 and 8 bits per pixel;  16 and 256 display colors)

���������������������������������������������������������������

�                           CAUTION                           �

�                                                             �

�  Although the #9GXi supports 4-color (2 bits per pixel)     �

�  operation, Windows does not.  You should NOT run Windows   �

�  or the Windows SETUP program when TIGA is set to 4-color   �

�  operation.                                                 �

�                                                             �

���������������������������������������������������������������

������������������������ͻ

�                        �

�  Windows Driver Files  �

�                        �

������������������������ͼ

The files on the #9GXi Windows Setup Disk are:

Filename      Description

��������      �����������

#9HAWKW.DOC   Documentation for the HawkEye feature set

#9WIN3 .DOC   GXi installation instructions for Windows 3.0

(this README file)

#9WIN31.DOC   GXi installation instructions for Windows 3.1

8514FIX.FON   Large-sized fixed fonts for Hi-Res driver

8514OEM.FON   Large-sized OEM fonts for Hi-Res driver

8514SYS.FON   Large-sized system fonts for Hi-Res driver

CCURSOR.EXE   The "Chameleon Cursor" Windows application

COURF  .FON   High resolution Times Roman fonts

GXI    .DLL   Library of shared Windows code

GXISETUP.EXE  #9GXi Windows Setup program

GXISETUP.INF  GXI Setup control file

HAWKEYE .EXE  The "HawkEye" Windows application

HELVF.FON     High resolution Helvetica fonts

HWZOOM.RLM    34020 Zooming RLM

METER.DLL     Dynamic Link Library used by #9GXi Windows Setup

program

OEMSETUP.INF  Information needed by the Microsoft Windows 3.1

Setup program

REXCHANGE.EXE The "Resolution Exchange" Windows application

TIGA.DRV      Multiresolution TIGA driver for Windows 3.1

TIGA.GR3      Enhanced Mode grabber for Windows 3.0

TIGATC.DRV    True color TIGA driver for Windows 3.1

TIGA48.DRV    Driver for 1280 and 1024 resolutions

(4 or 8 bit pixels) for Windows 3.0

TIGA48LO.DRV  Driver for 800 and 640 resolutions

(4 or 8 bit pixels) for Windows 3.0

TIGAOEM       Windows TIGA OEM diskette identification file

TIGAWIN.RLM   Coprocessor Windows engine (mapped color)

TIGAWINT.RLM  Coprocessor Windows engine (true color)

TMSRF.FON     High resolution Courier fonts

VDDTI30.386   Enhanced Mode virtual device driver for Windows 3.0

VDDTI31.386   Enhanced Mode virtual device driver for Windows 3.1

VGACOLOR.2GR  Standard mode grabber for Windows 3.1

VGADIB.3GR    Enhanced Mode grabber for Windows 3.1

VGALOGO.LGO   Windows logo information file

VGALOGO.RLE   Windows logo bitmap file

VGASYS.FON    Fonts for DOS window (in Windows 3.0)

WIN_AI.DLL    Windows Application DLL

ZMHOOK.DLL    Required by HAWKEYE.EXE

����������������������������������������ͻ

�                                        �

�  #9GXi Windows Setup Program Overview  �

�                                        �

����������������������������������������ͼ

Number Nine designed GXISETUP.EXE, the #9GXi Windows Setup Program

for fast, easy installation of the high-resolution #9GXi Windows 3.0

TIGA driver.

The GXISETUP program:

���������������������

# copies all the TIGA Windows 3.0 driver files from the #9GXi

Windows Setup Disk to your Windows 3.0 System directory

# backs up the original SYSTEM.INI and WIN.INI files to SYSTEM.VGA

and WIN.VGA

# modifies the SYSTEM.INI file to use the Windows 3.0 TIGA driver

# excludes the #9GXi TIGA communications buffer from the Windows

Expanded Memory pool

# modifies the WIN.INI file to automatically load the "Hawkeye"

application (Hawkeye must be installed before you can use the

virtual screen)

# creates the "HawkEye" program group

Before you install the #9GXi Windows 3.0 TIGA display driver:

�������������������������������������������������������������

#  Windows 3.0 MUST be installed and running on the #9GXi's

on-board VGA, an external VGA, or other display adapter

in your system.

#  Your #9GXi graphics board must be properly installed and

configured.

#  TIGA 2.0 must be loaded and configured.

#  TIGA resolution set (using the #9GXi utilities) to the

resolution at which you want to run Windows.  (You should

not select a Virtual Resolution before the #9GXi Windows

driver installation has been completed.)

(For more information, refer to Chapters 2 and 3 in your #9GXi

User's Installation Guide.)

System Requirements:

��������������������

#  TIGA Version 2.0 (or later).

#  Microsoft Windows 3.0.

#  1 MB of hard disk space for the #9GXi Windows 3.0 TIGA

driver files (plus whatever you need for Windows).

�����������������������������������������ͻ

�                                         �

�  Using the #9GXi Windows Setup Program  �

�                                         �

�����������������������������������������ͼ

To install the Windows 3.0 TIGA driver:

1 #  Start Windows.

The Program Manager is displayed.

2 #  Select 'Run' from the Program Manager 'File' menu.

The Run dialog box is displayed.

3 #  In the Command Line box, type:

A:GXISETUP.EXE

4 #  Select OK.

GXISETUP displays a confirmation window.

5 #  Select OK to run the GXISETUP program.

GXISETUP displays a message window indicating that the

Windows driver files are being copied.

Select 'Cancel' at any time to cancel installation.

��������������������������������������������������

�                      NOTE                      �

�                                                �

�  If you cancel driver installation, Windows    �

�  remains configured with the original driver.  �

��������������������������������������������������

Two message windows will be displayed when installation is

complete.  One indicates that the #9GXi Windows 3.0 TIGA

driver installation is complete.  The other indicates that

the "HawkEye" program group had been created.

6 #  Select OK.

The original Windows driver is still in memory, so you must:

7 #  Exit Windows to the DOS prompt.

8 #  Restart Windows to use your newly-installed Windows 3.0 TIGA

display driver.

��������������������������������������������������

�                      NOTE                      �

�                                                �

�  After installation, the original Windows      �

�  configuration can be restored by copying      �

�  SYSTEM.VGA to SYSTEM.INI and WIN.VGA to       �

�  WIN.INI.                                      �

��������������������������������������������������

Updating the #9GXi Windows TIGA Driver

��������������������������������������

Use GXISETUP.EXE (the #9GXi Windows setup program) to update an

earlier version of the #9GXi Windows TIGA driver with a new release.

You can NOT use the Microsoft Windows SETUP program to update your

driver because of a bug in the SETUP program that prevents copying

of new releases of a software driver over previous versions.

Follow the steps (above) in "Using the #9GXi Windows SETUP Program"

to update an earlier version of the #9GXi Windows driver.

���������������������������������������������ͻ

�                                             �

�  Using the Microsoft Windows SETUP Program  �

�                                             �

���������������������������������������������ͼ

You can use the Microsoft Windows SETUP Program only for the initial

installation of the #9GXi Windows 3.0 TIGA driver.  See the directions

below.  (Refer to the Microsoft Windows User's Guide for information

on running SETUP.)

�����������������������������������������������������������������

�                              NOTE                             �

�                                                               �

�  Although the Windows SETUP program will install the #9 GXi   �

�  TIGA Windows driver, it will not install the GXi "HawkEye"   �

�  application.  We recommend that you use GXISETUP (the #9GXi  �

�  Windows setup program) to install the TIGA Windows Driver.   �

�����������������������������������������������������������������

���������������������������������������������������������������

�                             CAUTION                         �

�                                                             �

�  The Microsoft Windows Setup program MUST be run from DOS.  �

�  Do NOT run the program within Windows or a DOS window.     �

���������������������������������������������������������������

To install the Windows 3.0 TIGA driver with the Windows SETUP program:

1 #  Start the SETUP program from DOS.

2 #  At the Windows SETUP screen, under System Information, move the

highlight to the "Display:" line.  Press [ENTER].

Windows SETUP presents a list of display adapters.

3 #  Move the highlight to the bottom of the display adapter list to

"Other (Requires a disk supplied by a hardware manufacturer)".

Press [ENTER].

4 #  When prompted by Windows SETUP to install the display driver disk,

insert the #9GXi Windows Setup disk in the disk drive.  Then press

[ENTER].

Windows SETUP reads the files on the disk and displays the

following list of TIGA display drivers:

TIGA Color Hires (1024 x 768 or greater)

TIGA Color Medium Resolution

TIGA Monochrome (1024 x 768 or greater)

5 #  Select a TIGA display driver:

TIGA Color Hires

(for 1280 x 1024 or 1024 x 768 resolutions)

TIGA Color Medium Resolution

(for 800 x 600 or 640 x 480 resolutions)

TIGA Monochrome

(for single-color [black and white] resolutions)

6 #  SETUP copies the Windows 3.0 TIGA driver files to the Windows

System directory.

�����������������ͻ

�                 �

�  Helpful Hints  �

�                 �

�����������������ͼ

This section offers advice on solving problems you might encounter

while using Windows or TIGA with the #9GXi.

TIGA Not Installed

������������������

When you work with Windows, TIGA can be inadvertently uninstalled.

The Windows 3.0 TIGA driver will not work if TIGA is not installed.

To check if TIGA is installed, use the #9GXi utilities to run the TIGA

demonstration program.  If the demonstration fails, reload the TIGA

Communications Driver (TIGACD).  At the DOS prompt,type:

[path]TIGACD -I

For [path], substitute the path to your TIGA directory.

There is no need to uninstall TIGACD before reloading it.  See Chapter 3

in the #9GXi User's Installation Guide for more information.

TIGA Memory Conflicts

���������������������

Sometimes there are conflicts between Windows and memory-resident

software.  These conflicts will often appear when Windows is used

on a VGA system also.

When it appears that TIGA Windows is not working, try running Windows

in VGA.  If the problem persists, unload other memory-resident programs

and restart Windows.

Windows Smart Drive Manager Conflicts

�������������������������������������

Windows 3.0 can operate in three modes: real, standard, and enhanced.

The mode that will be used is determined by computer type and memory

size. Often, a problem encountered in one mode does not appear in

another mode.

To run in:         Type:

����������         �����

real mode          WIN /R

standard mode      WIN /S

enhanced mode      WIN /3

The operation of many nonstandard disk-partitioning schemes conflicts

with the Windows 3.0 SMARTDRV.SYS disk manager.  This problem exists

on VGA Windows and TIGA Windows.

In enhanced mode, DOS memory managers often conflict with the TIGA

memory buffer.  Windows allows the TIGA memory to be excluded by using

the EMMEXCLUDE statement in the Windows SYSTEM.INI file.  (See the

SYSINI2.TXT file for more information about SYSTEM.INI.)  Number Nine

recommends that you add the following line to the [386Enh] section:

EMMEXCLUDE=C000-EFFF.

This tells Windows not to use the specified range of addresses. You can

experiment with this option.  For example, if you have a 386 system with

a VGA monitor running in enhanced mode, the statement might read:

EMMEXCLUDE=B800-C7FF

For a Novell Ethernet Network card at CC00:

EMMEXCLUDE=CC00-CFFF.

If you use the #9GXi's on-board VGA, try the following line to prevent

use of the CGA and VGA BIOS Area:

EMMEXCLUDE=B800-C7FF

If you mapped the #9GXi into the B000 address range, try this line:

EMMEXCLUDE=B000-B7FF.

���������������������������������������������������������������

�                              NOTE                           �

�                                                             �

�  If you change the TIGA Communications buffer you may have  �

�  to change the EMMEXCLUDE buffer (in the SYSTEM.INI file)   �

�  to match it.                                               �

���������������������������������������������������������������

��������������������ͻ

�                    �

�  Application Note  �

�                    �

��������������������ͼ

On certain systems, Windows Real Mode may hang when returning from

a DOS application.  Contact Number Nine Customer Support at

(617) 674-0009

if you encounter this problem.

�����������������������������������������������������ͻ

�                                                     �

�  #9GXi  (C) 1991, Number Nine Computer Corporation  �

�                                                     �

�����������������������������������������������������ͼ