Windows 3.1 TIGA Driver Installation

for the Number Nine GXi board

����������������ͻ

�                �

�  Introduction  �

�                �

����������������ͼ

Every Windows workstation must have a Windows driver that

matches its display adapter.  Systems that use the industry-

standard Texas Instruments Graphic Architecture (TIGA) use

drivers that take advantage of TIGA's special capabilities.

Number Nine Computer Corporation supplies a Windows 3.1 TIGA

driver that optimizes Windows performance on the #9GXi.

This file shows you, step-by-step, how to install the #9GXi

Windows 3.1 TIGA driver, so Windows 3.1 will use the #9GXi.

��������������ͻ

�              �

�  Background  �

�              �

��������������ͼ

Microsoft Windows 3.1 comes with its own setup program.  This

can install Windows on any of the more common "standard" display

adapters (such as VGA) available today.  It can even install for

generic versions of TIGA (though these can't use the GXi's own

special capabilities).

To make it easy to install a driver that takes full advantage

of the #9GXI, Number Nine has developed the GXISETUP Windows

application.  This application installs an optimized driver for

the GXi board and copies over files for the HawkEye application

suite.  This application is designed to work when either the

standard Microsoft TIGA driver is installed or if the Microsoft

VGA driver is installed.

�����������������������ͻ

�                       �

�  System Requirements  �

�                       �

�����������������������ͼ

#  an 80286 (or higher) CPU with at least 2MB of RAM.

A 80386 CPU is needed to use the true color windows

driver.

#  1 megabyte of available hard disk space for the GXi

TIGA driver files (plus the space you will need

for Windows itself).

#  a Number Nine GXi-family display adapter.

#  TIGA Version 2.05 (or later).

#  the Number Nine GXi Windows 3.1 TIGA drivers disk.

#  a set of Microsoft Windows 3.1 distribution disks.

����������������ͻ

�                �

�  Installation  �

�                �

����������������ͼ

Slightly different installation procedures may apply, depending

on what already exists on your system.  The scenarios are:

1 #  You do not have Windows on your system.

2 #  You have previously installed Windows 3.0, and are upgrading

to Windows 3.1.

3 #  You have Windows 3.1 already installed for a VGA, or for

"generic" TIGA.

In any case, before installing the GXi Windows driver, you should

install the GXi board and TIGA in your system.  Verify that TIGA

and the GXi are working properly by running the demo routine in

the GXI utility program.  Set the TIGA resolution to what you will

be using for Windows (use the GXI utilities program for this too).

Refer to Chapters 2 and 3 in your #9GXi User's Installation Guide

if you need more information on installing the GXi and TIGA.

�����������������������������������������������������������������

�                            CAUTION                            �

�                                                               �

�  Be sure TIGA is loaded and working when you install Windows. �

�����������������������������������������������������������������

��������������������������������������������������Ŀ

�  Installing from scratch (no previous Windows).  �

����������������������������������������������������

Installing Windows 3.1 for the GXi from scratch involves two main

steps:

1 #  Installing Windows for generic TIGA, using the Microsoft

Windows SETUP program.

2 #  Installing the GXi driver and "HawkEye" using Number Nine's

GXISETUP program.

To install Windows 3.1 for generic TIGA:

����������������������������������������

To install Windows 3.1 using Microsoft Windows SETUP, simply

follow the directions in the Microsoft "Getting Started" manual.

Briefly summarized, these are:

1 #  Insert Disk #1 (of the Windows 3.1 distribution set) into

an appropriate floppy drive.  Make that drive current.

2 #  Type "SETUP".  This starts the Windows setup program.

3 #  Follow the instructions presented on the screen.  Some

helpful hints here:

#  We generally recommend using "Express" setup.  You

will know if you need to use "Custom" setup (as you

should know what you're doing in that case).

#  Since TIGA is active when you run SETUP, Windows will

use TIGA as the default display device.  (If you

chose "Express" setup, it will not tell you it is

using TIGA, but that it is becomes obvious later.)

������������������������������������������������������

�                       NOTE                         �

�                                                    �

�  If for some reason TIGA were not loaded before    �

�  Windows installation began, Windows SETUP would   �

�  use whatever display was active -- likely the VGA �

�  onboard the GXi.  This is OK, since this display  �

�  is just temporary anyway until the GXi's HawkEye  �

�  driver is installed.                              �

������������������������������������������������������

#  On a 386 machine, you MUST NOT restart Windows from the

SETUP program, at the end.  Instead, return to DOS and

rerun Windows in Standard mode by typing:

WIN /S

The reason for this is to bypass a potential problem

of memory conflicts with TIGA in WINDOWS enchanced mode.

Please see the troubleshooting section for more

information.

������������������������������������������������������

�                      CAUTION                       �

�                                                    �

�  DO NOT restart Windows from SETUP on a 386.  You  �

�  MUST exit to DOS instead, and restart Windows in  �

�  Standard mode.                                    �

������������������������������������������������������

#  If you are using a resolution greater than 1024 pixels

horizontally (as is most likely), you should choose a

"large fonts" option.  The small fonts were designed

for lower resolutions, and may be hard to read at the

higher resolutions.

To install the HawkEye feature set:

�����������������������������������

Once you have installed Windows 3.1 for generic TIGA, all that

remains is to install the GXi driver and "HawkEye" application.

This enables advanced features (like hardware zoom and pan)

that are missing from the generic Windows TIGA drivers.

The discussion below assumes that you are somewhat familiar

with how to use Windows.  If you are not, you probably should

run the Windows and/or mouse tutorials when you first install

Windows.  Windows SETUP will offer to do this for you.

To install the GXi driver and "HawkEye":

1 #  Start up Windows if it is not already running.  (Do this

by typing "WIN /S" at the DOS prompt.)

2 #  Activate the "File Manager" program.  (This is one of the

standard applications distributed with Windows.  It will

appear in the "Program Manager" group of icons.)

3 #  Run the "GXISETUP.EXE" program on the Number Nine GXi TIGA

Windows Driver distribution diskette.  (This is done from

the "FILE" menu.)

4 #  Follow the onscreen instructions.

�����������������������������������������������������������������

�                            NOTES                              �

�                                                               �

�  While drivers written for Windows 3.0 will work with Windows �

�  3.1, the drivers written for Windows 3.1 contain performance �

�  enhancements not allowed in the Windows 3.0 environment.     �

�  The Windows 3.0 drivers are provided for those customers who �

�  prefer the Windows 3.0 environment.                          �

�                                                               �

�  If you cancel GXISETUP Windows 3.1 TIGA driver installation  �

�  then Windows will remain configured for the original driver. �

�����������������������������������������������������������������

����������������������������������������������Ŀ

�  Upgrading from Windows 3.0 to Windows 3.1.  �

������������������������������������������������

No unusual procedures are involved here.  You MUST use GXISETUP

to install HawkEye.  This reinstalls the #9GXi Windows driver.

Follow instructions in the Microsoft "Getting Started" manual for

installing Windows 3.1 for generic TIGA.  Then use the GXISETUP

application to install HawkEye.

���������������������������������������������������������������

�                            CAUTION                          �

�                                                             �

�  You MUST use GXISETUP to install the #9GXi "HawkEye" TIGA  �

�  driver for when upgrading to Windows 3.1.                  �

���������������������������������������������������������������

������������������������������������������������������Ŀ

�  Upgrading Windows 3.1 from another display driver.  �

��������������������������������������������������������

If you already have Windows 3.1 installed for some other display,

and wish to reinstall for the GXi, you can take a shortcut.  Go

directly to the procedure outlined above for installing "HawkEye",

and do it.  It is not neccessary to install Windows in this case.

�����������������������������������������������������������������

�                             NOTE                              �

�                                                               �

�  In order to switch back to the original TIGA display, the    �

�  should request that Windows reload the original files from   �

�  the Windows disks.  Once the original TIGA display driver    �

�  is loaded, applications within the HawkEye feature set can   �

�  not be run.                                                  �

�����������������������������������������������������������������

��������������ͻ

�              �

�  APPENDICES  �

�              �

��������������ͼ

���������������������������������������Ŀ

�  The GXISETUP Program:  What it Does  �

�����������������������������������������

The GXISETUP program is a Windows application developed by Number

Nine Computer Corporation.  Its purpose is to install the Number

Nine driver for the GXi board in Windows.  Number Nine's Windows

driver takes full advantage of the GXi's special features (such

as hardware zoom and pan), unlike the generic TIGA drivers that

come with Windows.

GXISETUP is run as an application from within Windows.  It:

#  copies all TIGA Windows 3.1 driver files from the #9GXi

Windows Setup Disk to your Windows 3.1 system directory;

#  backs up the original SYSTEM.INI and WIN.INI files to

SYSTEM.BOX and WIN.BOX (if a generic TIGA display) or

SYSTEM.VGA and WIN.VGA (if a VGA display);

#  modifies the SYSTEM.INI file to use the #9GXi Windows 3.1

TIGA driver;

#  excludes the #9GXi TIGA communications buffer from Windows'

expanded memory pool (setting EMMEXCLUDE in the SYSTEM.INI

file);

���������������������������������������������������������������

�                            CAUTION                          �

�                                                             �

�  You MUST use GXISETUP to install the #9GXi "HawkEye" TIGA  �

�  driver for Windows 3.1.                                    �

���������������������������������������������������������������

�����������������������������������������������������������Ŀ

�  Using Microsoft Windows SETUP to install the GXi driver  �

�������������������������������������������������������������

You must use Number Nine's GXISETUP program to install the GXi

driver.

�������������������������������Ŀ

�  #9 GXi TIGA Display Drivers  �

���������������������������������

There are four Windows TIGA drivers on the #9GXi Windows Setup

disk.  Driver resources (such as cursors, icons, etc.) vary,

based on display resolution and the number of displayed colors.

Filename      Used by

��������      �������

TIGATC.DRV    Windows 3.1 and "HawkEye", for true color.

TIGA.DRV      Windows 3.1 and "HawkEye", all other resolutions.

TIGA48.DRV    Windows 3.0, for horizontal resolutions of 1024

or 1280 pixels (at 4 or 8 bits per pixel, giving

16 or 256 colors).

TIGA48LO.DRV  Windows 3.0, for horizontal resolutions of 640

or 800 pixels (at 4 or 8 bits per pixel, giving

16 or 256 colors).

�����������������������������������������������������������������

�                            CAUTION                            �

�                                                               �

�  Though #9GXi supports 4-color (2 bit per pixel) operation,   �

�  Windows does not.  You should not run Windows or the         �

�  Windows Setup program if TIGA is set to 4-color operation.   �

�                                                               �

�  You MUST have set TIGA to a true-color resolution before     �

�  switching to the #9 HawkEye True-Color Driver in Windows.    �

�  The #9 HawkEye Resolution Exchange application is provided   �

�  to facilitate switching between a true-color and a non       �

�  true-color resolutions.                                      �

�����������������������������������������������������������������

��������������������������������������������Ŀ

�  Windows Driver Distribution Disk:  Files  �

����������������������������������������������

The files on the #9GXi Windows Setup Disk are described below.

(The list is in alphabetical order by filename.)

Filename      Description

��������      �����������

#9HAWKW.DOC   Documentation for the HawkEye feature set

#9WIN3.DOC    GXi installation instructions for Windows 3.0

#9WIN31.DOC   GXi installation instructions for Windows 3.1

(this README file)

8514FIX.FON   Large-sized fixed fonts for Hi-Res driver

8514OEM.FON   Large-sized OEM fonts for Hi-Res driver

8514SYS.FON   Large-sized system fonts for Hi-Res driver

CCURSOR.EXE   The "Chameleon Cursor" Windows application

COURF.FON     High resolution Times Roman fonts

DRIVER9.DLL   Library of shared Windows code

GXI.DLL       Library of shared Windows code

GXISETUP.EXE  #9GXi Windows Setup program

GXISETUP.INF  GXI Setup control file

HAWKEYE.EXE   the "HawkEye" Zooming Windows application

HAWKFILT.DLL  HawkEye Filtering Library (required by HAWKEYE

Feature set).

HELVF.FON     High resolution Helvetica fonts

HWZOOM.RLM    34020 zooming RLM

IMAGINE.FON   Enhanced #9 Dos Application Fonts.

METER.DLL     Dynamic Link Library used by #9GXi Windows Setup

program

OEMSETUP.INF  Information needed by the Microsoft Windows 3.1

Setup program

REXCHANG.EXE  The "Resolution Exchange" Windows application

TIGA.DRV      Multiresolution TIGA driver for Windows 3.1

TIGA.GR3      Enhanced Mode grabber for Windows 3.0

TIGATC.DRV    True color TIGA driver for Windows 3.1

TIGA48.DRV    Driver for 1280 and 1024 resolutions

(4 or 8 bit pixels) for Windows 3.0

TIGA48LO.DRV  Driver for 800 and 640 resolutions

(4 or 8 bit pixels) for Windows 3.0

TIGAOEM       Windows TIGA OEM diskette identification file

TIGAWIN.RLM   Coprocessor Windows engine (mapped color)

TIGAWINT.RLM  Coprocessor Windows engine (true color)

TMSRF.FON     High resolution Courier fonts

VDDTI30.386   Enhanced Mode virtual device driver for Windows 3.0

VDDTI31.386   Enhanced Mode virtual device driver for Windows 3.1

VGACOLOR.2GR  Standard mode grabber for Windows 3.1

VGADIB.3GR    Enhanced Mode grabber for Windows 3.1

VGALOGO.LGO   Windows logo information file

VGALOGO.RLE   Windows logo bitmap file

VGASYS.FON    Fonts for DOS window (in Windows 3.0)

WIN_AI.DLL    Windows Application DLL

�������������������ͻ

�                   �

�  Troubleshooting  �

�                   �

�������������������ͼ

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES              �

����������������������������������������������

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES 1            �

����������������������������������������������

�����������������������������������������������������������

� Windows cannot access your TIGA display adapter.        �

� The TIGACD program is not loaded, or is out of date.    �

� Windows requires TIGACD version 2.05 or higher.         �

� Please install your latest TIGA software, or use Windows�

� Setup to select VGA or another display type.            �

�����������������������������������������������������������

This WINDOWS error may be caused by TIGA not being installed

in your system or there may be a memory conflict with TIGA.

I. TIGA Not Installed

���������������������

The Windows 3.1 TIGA driver can't work if TIGA is not installed.

TIGA may not be installed or could have been inadvertently un-

installed.  To check, run the TIGA demonstration program.  If

the demonstration fails, the TIGA's Communications Driver

(TIGACD) can be reload by typing (at the DOS prompt):

[path]tigacd -i

For [path], substitute the path to your TIGA directory.  See

Chapter 3 in the #9GXi Installation Guide for more information.

The #9GXi INSTALL procedure modifies your AUTOEXEC.BAT file to

automatically install the TIGA driver when you turn on your

computer.  The AUTOEXEC.BAT may be failing to install the TIGA

driver.  Please check that TIGA is being installed by your

AUTOEXEC.BAT file.

If you have just installed an updated the version of the TIGA

driver from the #9GXi INSTALL disk then the TIGA driver would

have been uninstalled.  After installing the updated version

of TIGA you must reprogram your #9GXi graphics board.  To force

a reprogram of the board follow these steps:

1 #  Type GXI at the DOS prompt and select 'Use Advanced Utilities.'

The Advanced Utilities menu will appear.

2 #  Select 'Force GXi to Reprogram.'

The board will be reprogrammed and TIGA will be reinstalled.

II. TIGA Memory Conflicts

�������������������������

Sometimes Windows conflicts with a memory-resident program.  These

conflicts may also appear when other display drivers are used.  If

a problem is encountered with the TIGA Windows driver, the user

should try running the VGA Windows driver.  If the problem per-

sists, memory-resident programs should be unloaded until Windows

can successfully be restarted.

Windows SmartDrive Manager Conflicts

������������������������������������

Windows 3.1 can operate in two modes:  standard, and 386 enhanced.

The mode used is determined by computer type and memory size.  A

problem encountered in one mode often does not appear in another.

To Run In:         Type:

����������         �����

standard mode      win /s

enhanced mode      win /3.

Many nonstandard disk-partitioning schemes cause conflicts with

the Windows 3.1 SMARTDRV.SYS disk manager.  This may occur in

either VGA Windows or TIGA Windows.

In Enhanced Mode, DOS memory managers may conflict with the TIGA

memory buffer.  The TIGA buffer can be excluded from the memory

pool by using the EMMEXCLUDE statement in the Windows SYSTEM.INI

file.  (See the SYSINI2.TXT file for more information about the

SYSTEM.INI file.)

��������������������������������������������������������Ŀ

�                  NOTE                                  �

�                                                        �

�  The GXISETUP program does this for you automatically. �

����������������������������������������������������������

You can modify this.  For example, if your 386 system has a VGA

monitor, and you are running Windows in Enhanced Mode, the line

might read:

EMMEXCLUDE=C000-C7FF

For a Novell Ethernet Network card at CC00:

EMMEXCLUDE=CC00-CFFF.

If you mapped the #9GXi into the B000 address range:

EMMEXCLUDE=B000-B7FF.

���������������������������������������������������������Ŀ

�                            CAUTION                      �

�                                                         �

�  If ever you change the TIGA communications buffer, you �

�  should also modify EMMEXCLUDE in the SYSTEM.INI file to�

�  match the new TIGA buffer address.                     �

�����������������������������������������������������������

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES 2            �

����������������������������������������������

�����������������������������������������������������������������

� Windows cannot access your TIGA display adapter.              �

� TIGAWIN.RLM is missing from the Windows SYSTEM subdirectory.  �

� Please run Windows Setup to reinstall the TIGA display driver.�

�����������������������������������������������������������������

The TIGA WINDOWS driver has become corrupted.  Run the WINDOWS

Setup program from DOS.  After installing the driver run

the GXiSetup program to reinstall the HawkEye driver.

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES  3           �

����������������������������������������������

�����������������������������������������������������������������

� In order to use a pseudo color resolution in Windows, you     �

� must switch to a pseudo color resolution with the HawkEye     �

� Resolution Exchange.  Before you can start Windows and use    �

� the Resolution Exchange, you need to change the #9GXi         �

� resolution to a true color resolution.                        �

�                                                               �

� At the DOS prompt, type GXi.  Select 'Change #9GXi            �

� Resolution' from the menu, then choose a resolution with      �

� 65K or 16M colors.                                            �

�����������������������������������������������������������������

The #9GXi graphics board is in True-Color or High Color resolu-

tion, but Windows is currently set to use a pseudo color driver.

This could have happened because the user switched Windows

drivers using the Windows setup program or because the user

switched resolutions using the GXI program.  In either case,

the user must change the current GXi resolution before Windows

can be started.

To switch resolutions, a user should always use the Resolution

Exchange program.  This program is loaded by GXiSetup and

appears in the #9 HawkEye program group.

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES 4            �

����������������������������������������������

�����������������������������������������������������������������

� In order to use a true color resolution in Windows, you       �

� must switch to a true color resolution with the HawkEye       �

� Resolution Exchange.  Before you can start Windows and use    �

� the Resolution Exchange, you need to change the #9GXi         �

� resolution to a pseudo color resolution.                      �

�                                                               �

� At the DOS prompt, type GXi.  Select 'Change #9GXi            �

� Resolution' from the menu, then choose a resolution with      �

� 16 or 256 colors.                                             �

�����������������������������������������������������������������

The #9GXi graphics board is in a pseudo color resolution, but

Windows is currently set to use a true color driver.  This

could have happened because the user switched Windows drivers

using the Windows setup program or because the user switched

resolutions using the GXI program.  In either case, the user

must change the current GXi resolution before Windows can be

started.

To switch resolutions, a user should always use the Resolution

Exchange program.  This program is loaded by GXiSetup and

appears in the #9 HawkEye program group.

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES  5           �

����������������������������������������������

�����������������������������������������������������������

�                Extremely Low on Memory                  �

�                                                         �

� There is not enough memory to convert all the program   �

� icons.  The icons which are not converted will appear   �

� black.  You can fix this by choosing FILE.PROPERTIES.   �

� from the program manager.                               �

�                    ������Ŀ                             �

�                    �  OK  �                             �

�                    ��������                             �

�����������������������������������������������������������

The WINDOWS Program Manager has fixed the size of memory

allocated to support program icons in a single group to 64K.

This limits the number of icons that can be supported in

each WINDOWS GROUP.  When the display resolution is set

for 256 colors, then 64 icons can be displayed in the group.

Since most people use groups with less than 64 icon, this

limitation is generally not encountered in 256 color mode.

However, when the display is set to High Color, the 64K

limitation allows only 32 icons to be displayed in the

group.  When the display is set to TRUE COLOR only 16 icons

can be displayed.  The icons beyond these limit are displayed

as pure black icons, and still access the proper Windows

application when clicked.

DO NOT try to fix this problem by choosing FILE.PROPERTIES

from the program manager.  This operation will cause another

insufficient memory error to occur and will delete the program

icon from the group.

The only way to solve this problem is to reduce the number of

program icons in the group.  Each group should be designed so

that the number of program icons do not excede the resolution

dependent icon limit.

��������������������������������������������Ŀ

�        WINDOWS ERROR MESSAGES  6           �

����������������������������������������������

�����������������������������������������������������������

�  ��������������������������������������������Ŀ         �

�  �               Group File Error             �         �

�  ����������������������������������������������         �

� Insufficient memory to perform this operation.          �

� Quite one or more Windows applications and try again.   �

�                    ������Ŀ                             �

�                    �  OK  �                             �

�                    ��������                             �

�����������������������������������������������������������

This error will appear when you try to excede the resolution

dependent icon limit for a group.  See WINDOWS ERROR MESSAGES 5

above.

�������������������������Ŀ

�  Windows 3.1 Anomalies  �

���������������������������

These are problems in Windows itself.  Number Nine Computer can

not fix Windows bugs, but to keep our customers productive we

will try to recommend workarounds.

1 #  Microsoft TIGA installation does not set EMMExclude Area.

When installing Windows 3.1 using Microsoft's SETUP,

Microsoft's generic TIGA driver is normally installed.

However, the SETUP program does not exclude the TIGA

communications buffer from the Windows EMM.  Thus, the

system may hang if Windows restarts in enhanced (386)

mode.

Workaround:

Windows will still run in standard mode.  (DO NOT restart

Windows from the SETUP program.  Return to DOS and type

WIN /S

at the DOS command prompt.)  While in Standard mode, the

user can run GXISETUP, which will repair the problem.

2 #  Microsoft's TIGA installation assumes a VGA register set.

(GXISETUP also makes the same assumption.)

If the user has a non-VGA secondary monitor, VDDTIGA.386

may not initialize correctly.  The user will then see the

message "Video Initialization Failed", when they try to

restart Windows.  Again, Windows runs OK in Standard mode,

though the DOS prompt may fail.

Workaround:

The GXi has a VGA adapter right there on the board.  It

makes sense to use it unless you have unusual requirements.

We suggest you get a VGA-compatible secondary monitor.

Another method is to use the Virtual Display Device, the

286 grabber, and the 386 grabber of the secondary monitor.

This requires that Windows be installed for the secondary

monitor. (That way the driver files are correctly decom-

pressed from the Windows disks.)  Modify file SYSTEM.INI

appropriately.

For example, to use a Hercules/MDA monitor for a secondary

screen, put the following lines in the SYSTEM.INI file:

386Grabber=HERC.3GR     (in [boot] section)

286Grabber=HERCULES.2GR (in [boot] section)

display=VDDHERC.386     (in [386enh] section)

��������������������������������������������������������������ͻ

�                                                              �

�  #9GXi (C) Copyright 1992, Number Nine Computer Corporation  �

�                                                              �

��������������������������������������������������������������ͼ