ENGINEERING RELEASE NOTES

LC2X BIOS Version 2.0B.02

Recipient

| Customer | BIOS Version | Perfoce Version String | Deliverables |
| --- | --- | --- | --- |
| S3 | 86C362 2.0B.02 (32K ROM) | 362_2.0B.02 | 362.ROM 362.EXE 362.VCD |

Products Supported

S3 Ò LC2X  86C362

Purpose

- Disabled compile option to do double clocked modes.  Modes that were previously double clocked are now single clocked.

# Build Procedure

Uses MASM6.11 but with link.exe, lib.exe and nmake.exe from VISUAL C++1.5

To compile the S3 generic BIOS:

- Get S3 core level you want from the following directory in Perforce.

- BIOS98\MAIN

- BIOS98\VESA\DDC_1_0

- BIOS98\VESA\VBE_2_0

- BIOS98\VESA\DPMS_1_0

- BIOS98\VESA\S3_1_0

- BIOS98\VGA\VGA_1_0

- BIOS98\FONT

- BIOS98\INC

- BIOS98\CHIP

- BIOS98\CHIP\360

- In directory BIOS98\CHIP\360, run “MAKEBIOS 392 32K” to clean up the files and create binaries

Result is 362.ROM, 362.EXE, and 362.VCD in 360 directory.

© Copyright 1996 S3 Incorporated. All rights reserved. If you have received this document from S3 Incorporated in electronic form, you are permitted to make the following copies for business use related to products of S3 Incorporated: one copy onto your computer for the purpose of on-line viewing, and one printed copy. With respect to all documents, whether received in hard copy or electronic form,  other use, copying or storage, in whole or in part, by any means electronic, mechanical, photocopying or otherwise, is not permitted without the prior written consent of S3 Incorporated, P.O. Box 58058., Santa Clara CA 95052-8058. S3 and True Acceleration are registered trademarks of S3 Incorporated. The S3 Corporate Logo, S3 on Board, S3 on Board design, S3d design, Vision968, Trio, Trio64, Trio64V+, Trio64UV+, ViRGE, ViRGE/VX, S3d, Scenic, Scenic/MX2, Scenic Highway, Sonic, Sonic/AD,  DuoView, Cooperative Accelerator Architecture, Streams Processor, MIC, Galileo, Native-MPEG, No Compromise Integration, No Compromise Acceleration and Innovations in Acceleration are trademarks of S3 Incorporated. Other trademarks referenced in this document are owned by their respective companies. The material in this document is for information only and is subject to change without notice. S3 Incorporated reserves the right to make changes in the product design without reservation and without notice to its users.

- Modes supported

| VESA Mode | Resolution | Refresh Rates |
| --- | --- | --- |
| 100h | 640x400x8 | 70hz |
| 101h | 640x480x8 | 60hz 72hz 75hz 85hz |
| 102h | 800x600x4 | 56hz 60hz 72hz 75hz 85hz |
| 103h | 800x600x8 | 56hz 60hz 72hz 75hz 85hz |
| 104h | 1024x768x4 | 43hz I 60hz 70hz 75hz 85hz |
| 105h | 1024x768x8 | 43hz I 60hz 70hz 75hz 85hz |
| 106h | 1280x1024x4 | 43hz I 60hz 75hz 85hz |
| 107h | 1280x1024x8 | 43hz I 60hz 75hz 85hz |
| 109h | 132x25 Text | 70hz |
| 10Ah | 132x43 Text | 70hz |
| 10Dh | 320x200x15 | 70hz |
| 10Eh | 320x200x16 | 70hz |
| 10Fh | 320x200x32 | 70hz |
| 110h | 640x480x15 | 60hz 72hz 75hz 85hz |
| 111h | 640x480x16 | 60hz 72hz 75hz 85hz |
| 112h | 640x480x32 | 60hz 72hz 75hz 85hz |
| 113h | 800x600x15 | 56hz 60hz 72hz 75hz 85hz |
| 114h | 800x600x16 | 56hz 60hz 72hz 75hz 85hz |
| 115h | 800x600x32 | 56hz 60hz 72hz 75hz 85hz |
| 116h | 1024x768x15 | 43hz I 60hz 70hz 75hz 85hz |
| 117h | 1024x768x16 | 43hz I 60hz 70hz 75hz 85hz |
| 118h | 1024x768x32 | 43hz I 60hz 70hz 75hz 85hz |
| 119h | 1280x1024x15 | 43hz I 60hz 75hz 85hz |
| 11Ah | 1280x1024x16 | 43hz I 60hz 75hz 85hz |
| 11Ch | 640x400x15 | 70hz |
| 11Dh | 640x400x16 | 70hz |
| 11Eh | 640x400x32 | 70hz |
| 120h | 1600x1200x8 | 48hz I 60hz 75hz 85hz |
| 121h | 1600x1200x15 | 48hz I 60hz 75hz |
| 122h | 1600x1200x16 | 48hz I 60hz 75hz |
| 128h | 320x200x24 | 70hz |
| 129h | 640x400x24 | 70hz |
| 12Ah | 640x480x24 | 60hz 72hz 75hz 85hz |
| 12Bh | 800x600x24 | 56hz 60hz 72hz 75hz 85hz |
| 12Ch | 1024x768x24 | 43hz 60hz 70hz 75hz 85hz |
| 12Dh | 1280x1024x24 | 43hz 60hz 75hz 85hz |
| 12Eh | 320x200x8 | 70hz |
| 131h | 320x240x8 | 72hz |
| 132h | 320x240x15 | 72hz |
| 133h | 320x240x16 | 72hz |
| 134h | 320x240x32 | 72hz |
| 135h | 320x240x24 | 72hz |
| 141h | 400x300x8 | 72hz |
| 142h | 400x300x15 | 72hz |
| 143h | 400x300x16 | 72hz |
| 144h | 400x300x32 | 72hz |
| 145h | 400x300x24 | 72hz |
| 151h | 512x384x8 | 70hz |
| 152h | 512x384x15 | 70hz |
| 153h | 512x384x16 | 70hz |
| 154h | 512x384x32 | 70hz |
| 155h | 512x384x24 | 70hz |
| 161h | 1152x864x8 | 60hz 70hz 75hz 85hz |
| 162h | 1152x864x15 | 60hz 70hz 75hz 85hz |
| 163h | 1152x864x16 | 60hz 70hz 75hz 85hz |
| 164h | 1152x864x32 | 60hz 70hz 75hz 85hz |
| 165h | 1152x864x24 | 60hz 70hz 75hz 85hz |
| 171h | 720x480x8 | 75hz |
| 172h | 720x480x15 | 75hz |
| 173h | 720x480x16 | 75hz |
| 181h | 720x576x8 | 75hz |
| 182h | 720x576x15 | 75hz |
| 183h | 720x576x16 | 75hz |

# Changes and Additions

## Version 2.0B.02		10/20/98

| File Name | Modified By | Changes |
| --- | --- | --- |
| CHIP\390_391\ S3VER.INC | Kaymann Woo | - Updated version number to 2.0B.02. |
| - CHIP\390_391\ 362.VCD | Kaymann Woo | - Updated version number to 2.0B.02. |
| - CHIP\390_391\ MAKEFILE | Kaymann Woo | - Disabled conditional compile option to do double clocked modes. |

## Version 2.0B.01		9/21/98

| File Name | Modified By | Changes |
| --- | --- | --- |
| CHIP\390_391\ CHIPUTIL.ASM | Kaymann Woo | - In Enable_VGA, slammed 3C2[0]=1, to always force color. - In Test_Extended_Video_Memory, fixed bug with error in detecting 64 bit SDRAM. - In Modify_Registers, added code to program memory timings for 32 bit SDRAM and SGRAM. - Fixed bug with 2 Meg memory detection. - In Modify_Registers, added work around for ALI motherboard. - Modified Load_Patch_Registers to program CRA1 with the value for the extendede mode patch table. - In Modify_Regs, changed the CR85 to 17h and CR7B to 60h. |
| - CHIP\390_391\ S3VER.INC | Kaymann Woo | - Updated version number to 2.0B.01. - Modified version number to handle a 40K or 32K ROM version. |
| - CHIP\390_391\ CLKPLL.INC | Kaymann Woo | - Change PLL value using MaxVco=500, MinVco=250 - Added new PLL values to support hi refresh rate. |
| - CHIP\390_391\ EXTMODE.INC | Kaymann Woo | - Added support 100hz for 640x480, 800x600, 10x7, 11x8, 12x10 modes. - Added support 85hz for 16x12 modes. - Updated timings to get errors for VESA modes down to under 7%. - Modified 132x43, mode 10Ah, timings to fix problem with wide border |
| - CHIP\390_391\ MAKE362.BAT | Kaymann Woo | - New file |
| - CHIP\390_391\ EXTMODE.INC | Kaymann Woo | - Changed chip ID to 86C362 - Added memory timing table for 32 SDRAM and SGRAM memory. - Removed the resetting of CR53=00. - Cleaned up code to remove code that wasn't needed. - Modified extended_regs table to clean CRA1=00h. - Cleared CRA1 in the Extended Register Table - Set SR28 to 1Dh clear up jitter. |
| - CHIP\360\ VCP.INC | Kaymann Woo | - Added memory timing table for 32 bit SDRAM and SGRAM memory. |
| - CHIP\360\ VESAVCP.ASM | Kaymann Woo | - Changed the CHIP ID to 86C362. - Updated the chip revision to rev. B. |

Resolved Issues

None at this time.

Unresolved Issues

- VBETEST.EXE fails at 4bpp modes.

Testing Procedures

- Verify jitter issue with double clocked modes, 1280x1024x8@75/85 Hz and 1600x1200x8@60/75/85 Hz.

On SGRAM, EDO and SDRAM board:

1)Boot

- Check memory type and size

- CR36h[7-5] : 2 or 4 or 8MB

- Verify MCLK =100MHz for SGRAM/SDRAM, and 66MHz for EDO

- SR10h and SR11h

- Verify PCI subsystem and subsystem vendor ID at CR81h-CR84h.

- Verify SignOn message..

- Verify version number.

- 2)Monitor Detection

- If no monitor attached, beep 1 long, 3 shorts. BIOS will force it to monochrome mode

- If a color monitor attached, Savage3D boots at mode 3.

- 3)Verify all modes.

- Use BOX.exe and RRI program to check all supported modes under DOS. See  mode table.

- Make sure there is no jittering

- 4)Verify standard VGA modes compability.

- Use CHECKIT or QAPLUS to test

- 8x14.exe TSR must be loaded before the VGA test.

- 5)Verify VGA functions

- 6)Verify VESA functions

- Run SciTech VBETEST.EXE

- 7)Verify DDC functions

- Check DDC support

- Use DDCR.EXE to dump DDC data block.

- The first 8 bytes should be “00 FF FF FF FF FF FF 00”.

- 8)Verify VESA  timings

- Use the timing board to measure all vesa modes.

- 9)Verify S3 functions

- Service 4F14h, Function 00h - Query S3/VBE Information

- Service 4F14h, Function 01 - Display Refresh Control

- Subfunction 00 - Set Refresh Rate for Mode

- Subfunction 01 - Get Refresh Rate for Mode

- Subfunction 02 - Query Refresh Rates for Mode

- Service 4F14h, Function 02, Mode List Services

- Subfunction 00 - Reserved  For Future Use

- Subfunction 01 - Query Mode Number

- Subfunction 02 - Query Mode List

- Subfunction 03 - Get Extended Mode Information

- Service 4F14h, Function 03, Display Control

- Subfunction 00 - Set Active Display

- Subfunction 01 - Get Active Display

- Service 4F14h, Function 04, Display Detect

- Service 4F14h, Function 07, TV Control

- Subfunction 00 - Set TV Configuration

- Subfunction 01 - Get TV Configuration

- BIOSEDIT

BIOSEDIT can work correctly on LC2X BIOS.

Software Workarounds

- None at this time.