CS4232/36 OS/2TM
Software OEM Reference Manual

Preliminary Revision 1.00

Covers Crystal Audio Drivers Version 1.51

Print Date and Time: September 9, 1996

PRELIMINARY

Software License Statement

The software described in this manual is protected by U.S. and international copyright laws. You may not copy the software for any purpose other than making archival copies for the sole purpose of backing-up the software for protection against loss. A separate Software License may be obtained from Crystal that allows reproduction and distribution of the software.

The software may not be used on two or more machines at the same time.

Limit of Liability

In no event will Crystal be liable to you for any consequential or incidental damages, including any lost profits or lost savings, even if a Crystal representative has been advised of the possibility of such damages, or for any claim by any party.

Some states do not allow the exclusion of limitation of incidental or consequential damages, so the above limitation or exclusion may not apply to you.

Information in this document is subject to change without notice.

© 1996 Crystal Semiconductor Corp.  All rights reserved.

Reproduction in any manner whatsoever without the written permission of Crystal Semiconductor Corporation is strictly forbidden.

OS/2 and Warp are registered trade marks of IBM Corporation.

Sound Blaster and Sound Blaster Pro are registered trade marks of Creative Labs.

AdLib is a registered trade mark of AdLib Corporation

All brand and product names mentioned herein are trademarks or registered trademarks of their respective holders.  Crystal Semiconductor Corporation disclaims any proprietary interest in trademarks and trade names other than its own.

# Table Of Contents

Table Of Contents

Chapter 1: Introduction

Chapter 2: Overview and Installation

Overview

Resource Data

Installation

Installing on top of previous installations

System Requirements

File List for Crystal Audio Install Disk

Crystal Audio for OS/2 Installation Procedure

Device Driver Command Line Options

Notes on Command Line Options

CWAUDIO.INI vs Command line options

CWVAUDIO.SYS /S Option

CWAUDIO.SYS /R Option

Using Audio in DOS and Win-OS/2 Sessions

Audio Concurrency Issues - OS/2, DOS, Win-OS/2

Plug and Play System Issues

Mixer Issues

MIDI Issues

# Chapter 1:
Introduction

This manual describes the software package supplied with the CS4232/36 audio codec and covers how to install and use these programs. The package includes a OS/2  driver and support programs. This manual was designed for technicians, programmers, and engineers who need reference information about the Crystal Semiconductor CS4232/36 OS/2 Software Kit.

This document provides generic information concerning the CS4232/36 OS/2 Software Kit. For specific information refer to the release notes for the relevant release.

This Software Kit can be used  directly on the CRD4236-1 Audio Card and other Crystal Semiconductor reference designs and evaluation boards, including, CRD4232-1, CRD4232-2, CRD4232-3, CRD4236-3 and CRD9233-1. This Software Kit will also work with CS4237B and CS4238B audio codecs but will not take advantage of the sound enhancement features of these parts.  For full details on the Crystal reference designs and evaluation boards, please refer to the relevant Crystal Data Sheet.

# Chapter 2:
Overview and Installation

This chapter describes the contents of the CS4232/6 OS/2 Software Kit and how to install the audio driver and support programs.

## Overview

The software contained in the CS4232/6 OS/2 Software Kit consists of setup/installation files, OS/2  drivers and DOS support programs.

## Resource Data

To operate correctly in a Plug & Play environment, the CS4232/6 must be loaded with appropriate resource data. This can be achieved in several ways, the most secure of which  is from an external EEPROM. Resource data and utilities are included on a separate disk.

Please refer to the Crystal codec data sheet and the RESOURCE.TXT file on the CS4232/6 Utility disk.

Many of the examples and screen shots in this document will vary according to specific resource data entries.

Resource data can be used to configure the Crystal codec for Plug & Play operation or to disable BIOS Plug & Play operation. On the CS4232/36 Utilities Disk, two batch  files, PNP36.BAT and NOPNP36.BAT can be found. Running these batch files will program the EEPROM with data which enables or disables BIOS Plug & Play operation respectively. Boards are generally shipped from Crystal Semiconductor with Plug & Play disabled, though this is subject to change.

Note:  If the EEPROM of a CS4232 is programmed, the system MUST be rebooted so that it goes through a power cycle.

## Installation

### Installing on top of previous installations

Prior to installing, check if the following lines exists in CONFIG.SYS:

BASEDEV=CS31CONF.SYS ...

BASEDEV=CS32CONF.SYS ...

DEVICE=C:\MMOS2\CSBSAUD.SYS ...

DEVICE=C:\MMOS2\VCSBSAUD.SYS BSAUD1$

DEVICE=C:\MMOS2\VCSGAMES.SYS BSAUD1$

If any of these entries are present, delete them prior to installing this product.

### System Requirements

Before you install Crystal Audio for OS/2, check to see if you have the version of MMPM/2 as shipped with OS/2 2.11, or later, installed on your system.  Use the SYSLEVEL command to determine which level of MMPM/2 you have installed.  The "Current CSD Level" field displays one of the following

Values

| UN0000 | The version of MMPM/2 as shipped with OS/2 2.1. An update is required. |
| --- | --- |
| UN49407 | The October 1993 CSD version of MMPM/2. An update is required. This is the same version for the OS/2 2.11 service refresh of February 1994. |
| XR03000 | The version of MMPM/2 as shipped with OS/2 Warp 3.0. No update is required. If an update is required, the MMPM/2 CSD can be obtained free from CompuServe or from the IBM OS/2 BBS. |

CompuServe: OS/2 Support Forum:

Library 17: mmpmcs.dsk

IBM BBS: (919) 517-0001

mmoscsd.dsk

### File List for Crystal Audio Install Disk

Crystal Audio Files

| BSAUDRES.DLL | OS/2 business audio support DLL. |
| --- | --- |
| CARDINFO.DLL | Crystal device support DLL. |
| CWAUDIO.HLP | Help file used during installation. |
| CWAUDIO.BIN | Audio device microcode. |
| CWAUDIO.INI | Audio settings. |
| CWAUDIO.SYS | Audio physical device driver. |
| CWCONFIG.SYS | Audio device configuration device driver. |
| CWVAUDIO.SYS | Audio virtual device driver. |
| CONTROL.SCR | Installation control file. |
| FILES.SCR | Installation file list. |
| READ.ME | This file. |
| AUDIOIF.DLL | Updated DLL for mixer support. |
| AMPMXMCD.DLL | Updated DLL for mixer support. |

IBM OPL3 Files

| OPL3.SYS | OS/2 audio physical device driver for IBM OPL3 FM MIDI. |
| --- | --- |
| OPL3.HLP | Help file used during installation of IBM OPL3 FM MIDI. |
| OPL3RES.DLL | IBM OPL3 device support DLL. |
| OPLREAD.ME | Documentation for IBM OPL3 FM MIDI installation. |

IBM MPU401 Files

| MPU401.SYS | OS/2 audio physical device driver for IBM MPU401 MIDI. |
| --- | --- |
| MPURES.DLL | IBM MPU401 device support DLL. |
| MPUREAD.ME | Documentation for IBM MPU401 MIDI installation. |

IBM Misc. Files

| MIDIPLAY.ICO | Desktop ICON for MIDI player. |
| --- | --- |
| CLOCK01.SYS | OS/2 ISA clock driver that supports the highres timer. |
| CLOCK02.SYS | OS/2 MCA clock driver that supports the highres timer. |
| TIMER0.SYS | OS/2 highres timer driver. |

### Crystal Audio for OS/2 Installation Procedure

To install device support, insert the Crystal Audio for OS/2 diskette in drive A: (or whichever drive Crystal Audio for OS/2 diskette is in).  Start the Multimedia Application Install program which is located in the Multimedia folder on the OS/2 desktop.  Once you are in the multimedia installation program, select Source Drive A: (or whichever drive Crystal

Audio for OS/2 diskette is in).

The installation program will show a two Crystal Audio devices in the Features column: CS4231 and CS4232/CS4236. Select the device that applies to the Crystal Audio device installed on your system.  Do NOT select more than one Crystal Semiconductor Audio device.

A check mark will appear next to the feature(s) selected.

Click the install button to proceed with the installation.  Read and follow the instructions provided by the installation program.  The following files will be moved onto your system, based on the devices selected :

If Crystal Audio is installed :

\MMOS2\DLL\CARDINFO.DLL

\MMOS2\DLL\BSAUDRES.DLL

\MMOS2\HELP\CWAUDIO.HLP

\MMOS2\CWAUDIO.SYS

\MMOS2\CWAUDIO.INI

\MMOS2\CWAUDIO.BIN

\MMOS2\CWVAUDIO.SYS

\OS2\CWCONFIG.SYS

\MMOS2\DLL\AUDIOIF.DLL

\MMOS2\DLL\AMPMXMCD.DLL

If IBM OPL3 is installed :

\MMOS2\OPL3.SYS

\MMOS2\HELP\OPL3.HLP

\MMOS2\DLL\OPL3RES.DLL

If IBM MPU401 is installed :

\MMOS2\MPU401.SYS

\MMOS2\DLL\MPURES.DLL

If either OPL3 or MPU401 are installed :

\MMOS2\INSTALL\MIDIPLAY.ICO

\OS2\BOOT\CLOCK01.SYS

\OS2\BOOT\CLOCK02.SYS

\OS2\BOOT\TIMER0.SYS

Based on your responses to input panels, several lines will be added to the OS/2 CONFIG.SYS file.  If you do not know how to respond to the input panels, it is best to NOT change the default settings and click the OK button.  The following describes the modifications made to your OS/2 CONFIG.SYS file if no settings are modified, based on the device selected :

BASEDEV=CWCONFIG.SYS /P:534 /I:5 /D:1,1 /SB:220 /M:NONE /FM:NONE /J:NONE /CD:NONE

DEVICE=C:\MMOS2\CWAUDIO.SYS /N:BSAUD1$ /L:8

DEVICE=C:\MMOS2\CWVAUDIO.SYS BSAUD1$

If either OPL3 or MPU401 are installed :

BASEDEV=TIMER0.SYS

DEVICE=C:\MMOS2\OPL3.SYS /P:388 /N:OPL31$

DEVICE=C:\MMOS2\MPU401.SYS /N:MPU4011$

### Device Driver Command Line Options

The following tables describe the various command line options and setting available for the Crystal Audio device drivers.

CWCONFIG.SYS    -   OS/2 audio configuration device driver.

| Option/Format | Description | Valid Values (* = Default Value) |
| --- | --- | --- |
| /A:<asic type> | ASIC Type (CS4231 only) | OPTi928, OPTi929, MAD16, GALLANT, *NONE |
| /P:<port> | Codec Port | *534, 608, E84, F44 (hex) |
| /I:<irq> | Codec IRQ | *5, 7, 9, 11, 15 (decimal) |
| /D:<pDMA,cDMA> | Playback,Capture DMA | 0, 1, 3 (*1,1) |
| /CS:<port> | Control Port | (*not required) |
| /SB:<port> | Sound Blaster Port | *220, 240, NONE |
| /M:<port,irq> | MPU Port MPU IRQ | 330, *NONE 7, 9, 11, 12, 15, *NONE |
| /FM:<port> | FM Port | *388, NONE |
| /J:<port> | Joystick Port | 100, 200, *NONE |
| /CD:<type,port> | CD-ROM Type CD-ROM Port (CS4231 only) | Sony, Mitsumi, Panasonic, *NONE 320, 330, 340, 360, *NONE |
| /DL:<option> | Download microcode | Force, No (*not required) |
| /IFM:<option> | Internal FM (CS4236 only) | *On, Off |
| /O | Override PnP settings | (*not enabled) |
| /V | Enable Verbose Display | (*not enabled) |

CWAUDIO.SYS     -   OS/2 audio physical device driver.  This driver supports all Crystal Audio devices.

| Option/Format | Description | Valid Values (* = Default Value) |
| --- | --- | --- |
| /N:<name> | Name of PDD | Supplied by install program (*BSAUD1$) |
| /X1:<value> | AUX1 unmute/gain | 0..31 (8 = 0dB) (*not enabled - muted) |
| /X2:<value> | AUX2 unmute/gain | 0..31 (8 = 0dB) (*not enabled - muted) |
| /L:<value> | LINE unmute/gain | 0..31 (8 = 0dB) (*not enabled - muted) |
| /R:<rate> | Enable Adj Intrpt Rate | 1..60 (interrupts/sec) (*not enabled) |
| /EXT | Preserve Mixer Input | (*not enabled) |
| /LCAPT:<source> | Line capture source | L (line), X1 (AUX1), MX (MixOut) |
| /M:<option> | Microphone Monitoring | 1, On (*not enabled) |
| /S:<value> | Inactivity power down | 1..5 minutes (*not enabled) |
| /V | Enable Verbose Display | (*not enabled) |

CWVAUDIO.SYS     -  OS/2 audio virtual device driver.  This driver supports all Crystal Audio devices.

| Option/Format | Description | Valid Values (* = Default Value) |
| --- | --- | --- |
|  | Name of PDD | Supplied by install program (*BSAUD1$) |
| /S | Skip DOS Envir Var Scan | (*not enabled) |

### Notes on Command Line Options

#### CWAUDIO.INI vs Command line options

There are two ways to supply CWAUDIO.SYS with options that override default  settings. The first method is by supplying command-line options in the  CONFIG.SYS file as specified above. The second method is by supplying entries  in the CWAUDIO.INI file in the \MMOS2 directory. The first method overrides  the second on an individual entry basis. Therefore, entries in CWAUDIO.INI  will be used only when that particular option is not specified in CONFIG.SYS.

#### CWVAUDIO.SYS /S Option

The CWVAUDIO.SYS driver will automatically set a 'BLASTER' DOS environment  variable to the proper audio hardware settings.  This is done by adding a  '@SET BLASTER=' line to the beginning of all DOS session's autoexec file.  This file is specified in DOS session's DOS_AUTOEXEC property.  The /S option  for CWVAUDIO.SYS will skip the automatic setting of this environment variable.

#### CWAUDIO.SYS /R Option

The CWAUDIO.SYS /R option will allow the user to adjust the device interrupt   rate.  This option should be used with caution.

### Using Audio in DOS and Win-OS/2 Sessions

To enable Audio for DOS and Win-OS/2 sessions, you must set DOS properties to the following.

| WIN_RUN_MODE | 3.1 ENHANCED COMPATIBILITY |
| --- | --- |
| AUDIO_ADAPTER_SHARING | Required or Optional |
| INT_DURING_IO | On |
| HW_TIMER | On |
| VIDEO_SWITCH_NOTIFICATION | On |
| VIDEO_8514A_XGA_IOTRAP | Off |
| VIDEO_RETRACE_EMULATION | Off |
| DPMI_MEMORY_LIMIT | 8 |

### Audio Concurrency Issues - OS/2, DOS, Win-OS/2

OS/2 MMPM/2 is designed such that, when an application is running in the foreground (has focus), then that application takes priority over others running in the background.  Background applications are put on hold

until they are placed in the foreground (have focus).

When you have a single audio device, it is not possible for two programs to use the device at the same time.  For example, if one application has the device configured for audio playback, and a second application also configures the device for audio playback, the application with the focus takes priority and the application in the background is placed on hold. This situation also exists for sampling rate and sample size configuration.

Audio device drivers for Windows, installed under Win-OS/2, attempt to use the audio device in all Windows sessions.  For Win-OS/2 sessions which do not use the audio device, you can set the AUDIO_ADAPTER_SHARING property

to "None".  This prevents the session from accessing the audio device, and enables the device to be used by other applications.

To display OS/2 online documentation pertinent to system audio, use view.exe.  Suggested viewing items are

VIEW /AUDIO

VIEW /MULTIMEDIA

VIEW /SOUNDS

### Plug and Play System Issues

The CS4232 and CS4236 are Plug and Play (PnP) compatible devices.  If the system contains a PnP BIOS, the command line options are ignored unless resource data is not available or /O override option is specified.

### Mixer Issues

As of this release, there is no generic OS/2 mixer application for Business  Audio.  Therefore, the volume control for devices attached to mixer LINE, AUX1, and AUX2 is established on the command line of the CWAUDIO.SYS driver.  Refer to section 5, CWAUDIO.SYS, for the format of the unmute and gain setting options.

These values for gain are set on each boot and cannot be altered during system operation.  The command line values represent a gain setting where 8 is equalto 0 dB gain.  Each increment is 1.5 dB.  Values greater than 8 will reduce the

volume and values less than 8 increase the volume.

The devices attached to LINE, AUX1, and AUX2 are system dependent and cannot be detailed here.  Refer to your system documentation for device configuration.

Note: These settings do not effect standard wave audio volume which is controlled by the OS/2 volume control ICON.

### MIDI Issues

This installation diskette allows for the optional installation of the IBM OPL3 and MPU401 MIDI drivers.  These drivers are provided to the user as a convenience. Your system may or may not support MIDI audio.  Please refer to your system documentation for information related to your system's ability to support MIDI audio.

The CS4236 Crystal Semiconductor Audio device supports an integrated FM synthesizer.  Therefore, it is recommended that you install the IBM OPL3 driver to make use of this feature.  To do so, check both the Crystal Audio and the IBM OPL3 FM Synthesis features during Multimedia Installation.