DC21X4 fullmac driver for Alpha  Windows NT [tm] 3.5

Version v3.1

12-Feb-1996

1. Copyright Notice:

Digital Equipment Corporation makes no representations that the

use of its products in the manner described in this publication

will not infringe on existing or future patent rights, nor do

the descriptions contained in this publication imply the

granting of licenses to make, use, or sell equipment or software

in accordance with the description.

Copyright Digital Equipment Corporation 1994, 1995, 1996.

All rights reserved.

Do not redistribute the driver files.

Intel is a trademark of Intel Corporation.

Windows and Windows NT are trademarks of

Microsoft Corporation.

Microsoft, MS, MS-DOS are registered trademarks of

Microsoft Corporation.

Digital Semiconductor, DSChip, DEC, DECnet, Digital, PATHWORKS,

ThinWire, and the DIGITAL logo are trademarks of Digital

Equipment Corporation.

All other trademarks and registered trademarks are the property

of their respective holders.

2. Overview:

This file explains how to install the Windows NT

driver for Alpha .  The driver is currently supported for

motherboards and option boards based on the Ethernet controller

DSChip 21040 and Fast Ethernet controller DSChip 21140.

3. Requirements:

A.  Hardware Requirements:

Alpha based computer with an EISA or PCI bus.

DSChip 2104x Ethernet or DSChip 21140-based Fast

Ethernet PCI or EISA adapter.

B.  Software Revision Requirements:

Windows NT Version 3.5 or later

4. Files and directories:

---------------------------------------------------------------------

INSTALL.DOC       This file.

---------------------------------------------------

!DEC4250.CFG      The configuration file used by the DE425 Ethernet

board EISA configuration utility (ECU)

---------------------------------------------------

DC21X4.SYS        The driver.

---------------------------------------------------

OEMSETUP.INF      The driver's setup file.

---------------------------------------------------------------------

5. Installing the Hardware Boards:

A. Installing the DSChip 21x4 Evaluation Boards:

Refer to the DSChip 21x4 PCI Evaluation Board

User's Guide.

B. Installing the DSChip 21040 EISA Adapter Card (DE425):

1. Plug the board in an EISA slot

2. Run the EISA System Configuration Utility

(ECU)

3. In the EISA System Configuration,

select the [Configure computer] option

4. Insert a floppy with the DE425 EISA configuration

file !DEC4250.CFG

5. In the main menu, select the [Save and exit] option

to complete the initialization.

The options for the DE425 in the !DEC4250.CFG file are:

1. For interrupt:

o Edge-Triggered Interrupt (default)

o Level-Triggered Interrupt (no sharing)

o Level-Triggered Interrupt (sharing)

2. Supported interrupt numbers are:

o  5

o  9

o 10

o 11

3. For cable types:

o AutoSense/AutoDetection (default)

o Twisted Pair (10BaseT)

o Twisted Pair - Full Duplex

o Twisted Pair - No Link Test

o BNC (10Base2)

o AUI (10Base5)

6. Installing the DC21X4 Driver on Windows NT:

1. From the Windows NT Main Menu, select the Control Panel icon.

2. From the Control Panel menu select the Network icon.

If a dialog box asks you "Do you want to install NT

Network?" select YES and Continue. Then place

the Windows NT CDROM in the CD-ROM reader.

Select the drive that matches the configuration of your system

and complete the install.

3. You will see several messages. To add a network adapter, select Yes

and then Continue

4. For each additional message, select continue.

5. From the Add Network Adapter dialog box, scroll through the menu to

select Other.

6. When the Insert Diskettes menu prompts you for a disk, insert the

DC21X4 driver installation diskette for Windows NT into drive A.

Then select OK.

7. Select your option

[DEC PCI Ethernet DSChip 21040             ]

[DEC PCI Ethernet DSChip 21041             ]

[DEC PCI Fast Ethernet DSChip 21140        ]

[DEC DSChip 21040 Evaluation Board         ]

[DEC DSChip 21041 Evaluation Board         ]

[DEC DSChip 21140 Evaluation Board         ]

[DEC multia's Ethernet Controller           ]

[DEC DE434 EtherWORKS Turbo PCI TP Adapter  ]

[DEC DE435 EtherWORKS Turbo PCI Adapter     ]

[DEC DE500 FastEtherWORKS 10/100 PCI Adapter]

[DEC DE425 EtherWORKS EISA Adapter          ]

from the Select OEM Option box, then select OK.

8. Modify the default distribution path in the Windows NT

Setup box to point to the correct drive and path, then click

on [Continue]

9. For PCI options, a Dialog box pops up to select the:

ConnectionType:

------------------------------------------------------------------------

DSChip 21040

-------------------------------------------------------------------------

o AutoDetect                    In AutoDetect mode the driver

checks first the Twisted Pair port.

If the Link Test fails the driver

checks the BNC port. If no carrier is

detected the driver switches to the AUI

port.

o Twisted_Pair (10BaseT)

o Twisted_Pair Full_Duplex      To connect to Full Duplex hubs

o Twisted_Pair No_Link_Test     To connect hubs not supporting

the Link Integrity Test

o BNC (10Base2)                 BNC connector

o AUI (10Base5)                 15_pin connector

------------------------------------------------------------------------

DSChip 21041

-------------------------------------------------------------------------

o AutoSense                     Senses dynamically the media ports

supported by the adapter. The precedence

is TP,BNC and AUI.

o AutoSense No_Nway             Same mode than Autosense without

Nway negotiation.

o Twisted_Pair (10BaseT)

o Twisted_Pair Full_Duplex      To connect to Full Duplex hubs

o Twisted_Pair No_Link_Test     To connect hubs not supporting

the Link Integrity Test

o BNC (10Base2)                 BNC connector

o AUI (10Base5)                 15_pin connector

------------------------------------------------------------------------

DSChip 21140\21140A

-------------------------------------------------------------------------

o AutoSense                     Senses dynamically the media ports

o AutoSense                     Senses dynamically the media ports

without Nway negotiation

o 100BaseTx                     To connect 100BaseTx Ethernet hubs

o 100BaseTx Full_Duplex         To connect 100BaseTx Full Duplex hubs

o 100BaseT4                     To connect 100BaseT4 Ethernet hubs

o 100BaseFx                     To connect 100BaseFx Ethernet hubs

o 10BaseT                       To connect 10BaseT Ethernet hubs

o 10BaseT Full_Duplex           To connect 10BaseT Full Duplex hubs

o BNC (10Base2)                 BNC connector

o AUI (10Base5)                 15_pin connector

10. Upon completion, the selected adapter is added to the

Installed Adapter Cards list of the Network Setting box

(the number prefixing the adapter is the adapter number).

11. Click on [Close] to complete the Network Setting

initialization.

12. If needed, modify the values in the registry (see section 7 below).

13. Shutdown and reboot the system.

7. Installing multiple PCI boards:

Repeat the procedure above.

8. Parameters

To overwrite the initial values set by the EISA Configuration

Utility (ECU) overwrite the driver's default values,

add or edit the following values in the Registry

DC21x4_<adapter_number> Parameters subtree:

NetworkAddress = REG_SZ { xx-xx-xx-xx-xx-xx }

Mandatory if no Serial ID PROM is plugged in

Overwrite the Serial ID PROM if plugged in

NetworkAddress 00-00-00-00-00-00 is ignored

ReceiveBuffers = REG_DWORD {8..64} (default 16)

Number of buffers allocated by the

driver in the Receive ring.

MapRegisters = REG_DWORD {4,8,16}   (default: 16(alpha) | 8)

Number of map registers allocated

by the adapter.

CacheLineSize = REG_DWORD { 16 | 32 | 64 | 128 }

16 bytes

32 bytes (default)

64 bytes

128 bytes

The following values allow to overwrite the DC21X4 default setting.

Refer to the DSChip 21040, DSChip 21041 & DSChip 21140 Engineering

specifications for detailled description:

BackPressure = REG_DWORD { 0 | 1 }  (not DC21040 rev1)

CSR6<BP>    0  Disabed (default)

1  Enabled

BurstLength = REG_DWORD  { 1 | 2 | 4 | 8 | 16 | 32 }

CSR0<PBL>  1  doubleword

2  doublewords

4  doublewords

8  doublewords

16  doublewords   (default)

32  doublewords   (not DC21040 rev1)

CaptureEffect = REG_DWORD { 0 | 1 }  (not DC21040 rev1)

CSR6<CA>    0  Disabed (Default)

1  Enabled

FifoArbitration = REG_DWORD { 0 | 1 }

CSR0<BAR>   0 Rcv/Txm Arbitration (default)

1 Round_Robin

SnoozeMode = REG_DWORD { 0 | 1 }  (DC21041 only)

CFDA<SM>    0 Disabled (default)

1 Enabled

StopBackoffCounter = REG_DWORD { 0 | 1 }  (not DC21040 rev1)

CSR6<SB>    0  Disabed (Default)

1  Enabled

StoreAndForward = REG_DWORD { 0 | 1 }  (DC21140 only)

Select the StoreAndForward mode if high

PCI latencies causes 100Mbps Transmit Underruns

CSR6<SF>    1 Enabled

0 Disabled (default)

TransmitThreshold = REG_DWORD { 72 | 96  | 128 | 160 }

Transmit Threshold value for the 10Mbps media

CSR6<TTM,TR>  72 bytes

96 bytes (default)

128 bytes

160 bytes

TransmitThreshold100 = REG_DWORD { 128 | 256 | 512 | 1024}

Transmit Threshold value for the 100Mbps media

Meaningless when StoreAndForward is set

CSR6<TTM,TR>  128 bytes

256 bytes

512 bytes (default)

1024 bytes

These Registry values can be edited interactively through the Registry

editor (winnt\system32\regedt32.exe) or by invoking the regini command

Edit an DC21X4.ini file as described below, then invoke the command:

regini DC21X4.ini

DC21X4.ini format:

\Registry\Machine

System

CurrentControlSet

Services

dc21x4_{adapter_number}

Parameters

{valuename} = {type}{value}

9. Error messages

The error messages are logged in the Entry Log and can be seen using the Entry Log Viewer (in

WNT's Administration Tools group).

Each error message has a standard Ndis error code and an driver's additional code.

The additional codes are:

DC21X4_ERRMSG_REGISTRY            0x01

DC21X4_ERRMSG_ALLOC_MEMORY        0x02

DC21X4_ERRMSG_SROM                0x03

DC21X4_ERRMSG_MEDIA               0x04

DC21X4_ERRMSG_LOAD_CAM            0x05

DC21X4_ERRMSG_SYSTEM_ERROR        0x06

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT  0x07

9.1 Driver initialization errors:

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_REGISTRY

The AdapterType Registry's key is missing or its value is

unsupported by the adapter board

o NDIS_ERROR_CODE_ADAPTER_NOT_FOUND

No board matching the AdapterCFID Registry's key value was found

plugged into the PCI bus or in the slot specified in the Registry

o NDIS_ERROR_CODE_OUT_OF_RESOURCES

DC21X4_ERRMSG_ALLOC_MEMORY

Not enough memory to allocate space for the Adapter data block

o NDIS_ERROR_CODE_INVALID_VALUE_FROM_ADAPTER

DC21X4_ERRMSG_SROM

Invalid data read from the adapter's serial ROM

o NDIS_ERROR_CODE_NETWORK_ADDRESS

No readable burnt_in or software configured station address

o NDIS_ERROR_CODE_UNSUPPORTED_CONFIGURATION

DC21X4_ERRMSG_MEDIA

The selected media port is not supported by the board.

o NDIS_ERROR_CODE_INTERRUPT_CONNECT

Interrupt initialization failed

o NDIS_ERROR_CODE_TIMEOUT

DC21X4_ERRMSG_LOAD_CAM

DC21X4 CAM initialization failed

9.2 Driver run time errors:

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_SYSTEM_ERROR

System_Error interrupt (bus parity error)

o NDIS_ERROR_CODE_HARDWARE_FAILURE

DC21X4_ERRMSG_TXM_JABBER_TIMEOUT

Transmit Jabber timer expired

10. Hotline and Support Information:

Call the Digital Semiconductor Information Line for information and

technical support:

United States and Canada              1-800-332-2717

Outside North America                +1-508-628-4760

Ordering Digital Semiconductor Products

To order Digital Semiconductor products, contact your local

Digital Semiconductor sales office.  When working with your sales

representative, you may be able to take advantage of discounts

and volume pricing.

To order Digital Semiconductor samples or for a complete list of

products call the Digital Semiconductor Information Line.

You can order the following network products from Digital

Semiconductor:

21040-AA      DS 21040  PCI Ethernet LAN Controller

21041-AA      DS 21041  PCI Ethernet LAN Controller

21140-AB      DS 21140  PCI Fast Ethernet LAN Controller

21140-AC      DS 21140A PCI Fast Ethernet LAN Controller

21A40-01      DS 21040 PCI Evaluation Board Kit

21A41-01      DS 21041 PCI Evaluation Board Kit

Ordering Associated Literature

Digital Semiconductor documentation and literature is available

with the products. For a complete list and for information about

ordering, contact the Digital Semiconductor Information Line.

Ordering Third-Party Literature

You can order the following third-party literature directly from

the vendor:

Title                       Vendor

----------------------------------------------------------

PCI System Design Guide    PCI Special Interest Group

N/S HH3-15A

5200 N.E. Elam Young Pkwy

Hillsboro, Oregon 97124-6497

1-503-696-2000

----------------------------------------------------------

Electronic Orders

To place an order through your account at the Electronic Store,

dial 1-800-234-1998, using a modem set to 2400- or 9600- baud.

You must use a VT terminal or terminal emulator set at 8 bits,

no parity. If you need assistance using the Electronic Store,

call 1-800-DIGITAL (1-800-344-4825) and ask for an Electronic

Store specialist.

Telephone and Direct Mail Orders

From        Call                Write

__________________________________________________________

U.S.A.      DECdirect           Digital Equipment Corporation

Phone:              P.O. Box CS2008

1-800-DIGITAL       Nashua, NH 03061

(1-800-344-4825)

FAX: 603-884-5597

Puerto      Phone:              Digital Equipment Corporation

Rico        809-781-0505        3 Digital Plaza, 1st Street

FAX: 809-749-8377   Suite 200

Metro Office Park

San Juan, Puerto Rico 00920

Canada      Phone:              Digital Equipment of Canada Ltd.

1-800-267-6215      100 Herzberg Road

FAX: 613-592-1946   Kanata, Ontario, Canada K2K 2A6

Attn: DECdirect Sales

International  -                Local Digital subsidiary or

approved distributor

Internal    DTN 264-3030        U.S. Software Supply Business

orders      603-884-3030        Digital Equipment Corporation

FAX: 603-884-9690   10 Cotton Road

Nashua, NH 03063-1260