MARCH 1994

Interactive V/386 R3.2 ( TCP/IP 1.3)

Driver Installation

for

DE-220 Series Ethernet Card

======================================

# dosget setup220 setup220

# chmod +x setup220

# ./setup220

# sysadm

* System Adminaction *

1. select " SOFTWARE "

and select " NETDRVR " of " SOFTWARE "

then select " CONFIGURE " of " NETDRVR "               choice " d22e0 "

( please check value of the IRQ, Start and End I/O address on I/O bus )

If so, type "Y". If not, type "N"

2. Do you wish to build a new kernal at this time ? (y. n. q.)   repeat while

adapter more

than one

** Kernel Configuration **

(1). Build

Build a Kernel ?                                Yes

Write a description of this Kernel              D-Link DE-220 * 1

(2). Install

(a). UNIX.n   D-Link DE-220 * 1

(b). INSTALL

Shutting down system

choice " CANCEL "

choice " QUIT "

* System Adminaction *

3. select " SOFTWARE "

and select " TCPIP "

then select " HOSTMGMT "

Enter the operation you want to perform :

1. list

2. add

3. delect

(default list) [q] : 2

Enter the domain name of the host you want to add : localhost

Enter the network address that will be used for the

connection to machine 'localhost'

Address: [?] 192.9.200.88

Enter the alias name of the host you want to add :    D-Link

Here is the hosts file entry for machine 'localhost'

System Name : localhost

Address     : 192.9.200.88

Alias       : D-Link

Should this be entered into the '/etc/hosts' file ? [y.n.q.]   y

Add another entry to the hosts file ? [y.n.q.]                 q

Enter the operation you want to perform [y.n.q.]               q

* System Adminaction *

4. select " SOFTWARE "

and select " TCPIP "

then select " HBTCP "

and select " ADDITERFACE "

** Additerface Command Menu **

1. Add a driver from the network driver subset as a new interface

2. Add a third party Ethernet driver as a new interface

3. Add a slip interface

4. Configure TCP/IP for local operation only ( no interface card )

5. List the installed and configrued drivers

q. quit

Enter the operation you want to perform :                         2

Enter the name of the driver you which to use : d22e0

Enter the hostname for this interface : localhost

The current netmask is : 255.255.255.0

Change the netmask ? [n] [y.n.?.]

The current broadcast address is :

Do you want to change the broadcast address ? [n] [y.n.?]           n

** Additerface Command Menu **

1. Add a driver from the network driver subset as a new interface

2. Add a third party Ethernet driver as a new interface

3. Add a slip interface

4. Configure TCP/IP for local operation only ( no interface card )

5. List the installed and configrued drivers

q. quit

Enter the operation you want to perform :                          q

You system is configured to use 192.9.200.201 as a gateway.

Do you want to change the gateway ? [n] [y.n.?]                    n

Press Enter to continuce

# cd /

# shutdown -y -g0 -i6