(C) COPYRIGHT HEWLETT PACKARD COMPANY 1996. ALL RIGHTS RESERVED.

No part of this program may be photocopied, reproduced or translated

to another program language without the prior written consent of the

Hewlett-Packard Company.

HPVGSet version 1.19

====================

Notation :

==========

[xxxx]  hexadecimal number with four digits (word)

[dd]    decimal number with 2 digits

[?]     character

[name]  a valid file name

Introduction :

==============

HPVGSet can be run in an interactive mode or a command line

mode.

All the options are case sensitive. This is a modification

from previous version of HPVGSet and may cause some problem for people

using preset batch files.

Also some of the old command line options have been removed and

are no longer supported. Please see the list of all the options in

the following sections.

Interactive Mode :

==================

HPVGSet is run in interactive mode if no options are specified on the

command line, or the only options specified are /z or /pci20.

The use of any other option(s) will force HPVGSet to be run

in command line mode.

To run HPVGSet in interactive mode.

Type [path]:HPVGSet

For PCI 2.1 machine that are known to have problem with PCI network

cards, use the PCI 2.0 scan algorithms.

Type  [path]:HPVGSet /pci20

When using the PCI 2.0 scan algorithms, the slot information

is not available.

To use HPVGSet in expert interactive mode, use the /z option.

Type  [path]:HPVGSet /z

The expert mode, as its name indicates, should only be use by users

that have a very good understanding of the card(s) settings.

Command Line Mode :

===================

The following is a list of all the options that are now available from

the command line.

The command line options can be thought of as a small set of

keywords.

To use HPVGSet command line mode

Type  [path]:HPVGSet [command line option]

/a[xxxx]  Select the card at the base I/O address <????>

as the target for the option following on the command

line, up to the next /a option.

If only one card is present, it is used as the

default card and the option is not required.

For multiple cards, the option is always required.

/b[xxxx]   Set the card base I/O address.

/c[?]      Choose the card connector.

/cs   Set the connector to auto select.

/cb   Set the connector to BNC.

/cr   Set the connector to RJ45.

/ca   Set the connector to AUI.

/cv   Set the connector to VG 100mb.

/d[xxxx]   Set the card data transfer mode.

[] = 0   set card to I/O mode.

[] > 0   set card to memory map at address <xxxx>.

/e[d]      Force the display of all the error messages.

/e0    Turn off all the display of all the error

messages.

/e1    Turn on the display of all the error messages.

By default the all the error messages are

displayed.

/f         Set the card to the factory settings.

/h /?      Display this help message.

/i[dd]     Set the card interrupt vector to <dd>.

/l[d]      Set the card to full or half duplex.

/l0   Set to half duplex.

/l1   Set to full duplex.

/m[d]      Set the card in bus master or slave mode.

/m0   Set the card to slave mode.

/m1   Set the card to bus master mode.

/o         Display the card's current settings.

/p[d]      Set the card plug and play mode.

/p0   Turn plug and play off.

/p1   Turn plug and play on.

/q         querry the system for all the cards

and display their settings

/r[name]   Read the option(s) from the file named <name>.

The option read from the file are inserted

sequentially at the place of the file name.

/s         Save the current ASIC settings to EEPROM.

/t[?]      Test the card.

/t    Perform all the following test <=>

/tr+/tm...+/tb.

/tr   Perform a register test.

/tm   Perform a memory test.

/td   Perform a DMA channel test.

/ti   Perform an interrupt test.

/ta   Perform a bus master test.

/tl   Perform a loopback test.

/tb   Perform a boot ROM test.

/w[name]   Write the current asic settings to the option(s) file

named <name>.

The command line options can be combined together to perform

multiple actions on a given card.

If multiple cards are present, one must be chosen using

the /a option.

Example :

=========

HPVGSet /a300 /b380 /i9 /dCC000 /s /wmyfile.cfg /o

The above command chooses the card with base I/O address

300H sets its new base address to x380, changes its irq to 8, set

its data transfer mode to memory map at address CC000H.

Note that the previous actions have only modified the ASIC.

To save the settings to the EEPROM, the /s option is used.

The /w option writes the current ASIC settings to

myfile.cfg.

The later file can be used for the /r option.

If multiple cards are present the following is valid

HPVGSet /a300 /o /aFE00 /o

This output the current settings for the cards at base

address 300H and FE00H.

Errors :

========

All the command line option return errors. In this version

of HPVGSet the error handling is comprehensive, but lacks "user

friendliness".

As such, a set of command lines should be tried with one

option at a time before grouping them into one line.

Also, some of the options may be legal for some cards and

not for other.

CMD0001 - Dynamic memory allocation error.

A possible fix is to unload the number of TSR

programs currently loaded in the system.

CMD0002 - Invalid parameter in function call.

CMD0003 - Invalid option or option argument.

An invalid option was specified on the command

line, or a given option is not available for the selected

network card.

Try all the options individually, (always including /a if

necessary) to identify the one causing the error.

CMD0004 - Invalid option format.

A command line option was specified with an invalid argument.

Look through the command line to identify the option.

CMD0005 - Invalid interrupt request number

The interrupt number specified with the /I option

is invalid.

Choose a number in [3,4,5,6,7,9,10,11,12,15].

CMD0006 - Interrupt request conflict.

The specified interrupt number is already in use.

Select another interrupt number.

CMD0007 - Error accessing config file.

With the /w option, means that the file could not

be created.

With the /r option, means that the file could not

be opened or that its content was corrupted.

CMD0008 - Card not found.

No card was found at the base I/O address

specified with the /a option, or no network cards

were found in the system.

CMD0009 - Invalid base I/O address.

The base I/O address specified with the /b option

is invalid.

Select an address in [100H - to 340H] in increment

of 40H.

CMD0010 - Error setting card to base I/O address.

The base I/O of the card was not successfully

changed.

Try a different base I/O address.

CMD0011 - Invalid memory map address.

The memory map specified with the /d option is invalid.

Select an address in [C8000H - EC000H] in increment of

4000H.

CMD0012 - Error setting the memory map address.

The memory map address was in use.

Try a different memory address.

CMD0013 - Error saving ASIC configuration to EEPROM.

The current ASIC settings could not be saved into

the EEPROM.

This is a major error and should be reported to

your HP authorized LAN dealer or HP representatives.

CMD0014 - Invalid bus master mode.

The selected card could not be set to bus master

mode.

Move your card to another slot that supports bus

mastering.

CMD0015 - Invalid factory setting.

The selected card could not be reset to the

factory defaults.

This is a major error and should be reported to

your HP authorized LAN dealer or HP representatives.

CMD0016 - Card memory test failure

The card's on-board RAM or the associated circuitry

failed during the card test.

Verify that the card is installed correctly.

CMD0017 - Card register test failure

Most likely due to an I/O address conflict.

Change the base I/O address range your card is using.

CMD0018 - Card interrupt test failure.

The current card interrupt setting conflicted with

existing software or hardware.

Choose another interrupt setting for the selected

card.

CMD0019 - Card bus master test failure

CMD0020 - Card loopback test failure

Interference to the test from a connected LAN

cable, or possible hardware failure.

Remove any LAN cable connected to the card.

CMD0021 - Card boot ROM test failure.

The boot ROM is not correctly installed in the

socket, OR the boot ROM's code may have become

corrupted.

CMD0022 - Invalid connector selection.

The connector specified with the /l option is not

available for the selected card.

Try another connector.

CMD0023 - No supported card found.

No known cards were found in system.