Evergreen 5x86 Register Bit Enabler

Install Disk Version 2.0

06/05/96

Files included on Evergreen 5x86 Install Disk:

DISK.ID		File used by install program to verify disk.

ETDIAG.BAT	Batch file used to run ETDIAG.EXE program from floppy.

ETDIAG.BAT	Batch file that is copied to root directory of install drive, used to run ETDIAG.EXE.

INSTALL.DAT	Script file used by INSTALL.EXE program.

INSTALL.EXE	Program used to install Evergreen 5x86 software drivers.

PUTLINE.EXE	Utility program used by INSTALL.EXE.

README.TXT	This file list and possible updated information.

\DOS

ET586.EXE - Software driver for DOS\WINDOWS environments.

\ETDIAG

ETDIAG.EXE	Diagnostic program used to identify CPU type, speed, etc. Use to confirm that cache is turned on.

ETPARTS.TXT	Text file containing Evergreen parts information used by ETDIAG.EXE.

README.TXT	Text file explaining how to run ETDIAG.EXE.

SOCKETS.TXT	Text file explaining various CPU socket styles, used by ETDIAG.EXE.

TESTCB.BAT	Batch file used to test Cache-Back (tm)/Write-Back Cache compatibility.

\NT  (Note: To install support for NT run SETUP.EXE from this subdirectory.)

README.TXT	Text file explaining how to install NT 5x86 driver and configuration program.

ET586NT.SYS	Driver file

ET586NT.EXE	Configuration program file.

MFC30.DLL		Used by configuration program.

MSVCRT20.DLL	Used by configuration program.

SETUP.EXE		Setup program file.

SETUP586.INF	Used by setup program.

Mscomstf.dll		Used by setup program.

Mscuistf.dll		Used by setup program.

Msdetstf.dll		Used by setup program.

Msinsstf.dll		Used by setup program.

Msshlstf.dll		Used by setup program.

Msuilstf.dll		Used by setup program.

\OS2

ET586OS2.SYS 	Software Cache driver for OS\2 2.x, 3.x, command line version.

Note: Device line needs to be last line of the CONFIG.SYS file.

Parameters are set as follows:

USAGE: DEVICE=ET586OS2.SYS [/CCR0=xx] [/CCR4=xx] [/WBE]

Where: xx is a two digit hex number (example: /CCR4=1D).

/PCR0=xx 	Sets performance configuration register 0

/CCR2=xx 	Sets custom configuration register 2

/CCR4=xx 	Sets custom configuration register 4

/CCD     	Clears cache disable bit in MSW

/WBE     	Enables write back cache on 586

/WBD     	Disables write back cache on 586

/CBE     	Enables Cache Back on 586, writes to IO port

Windows NT 4.0

Readme for Evergreen 5x86 Upgrades running Windows NT 3.5x - 4.x    06/04/96

Installing Driver

To install NT 5x86 driver and configuration program, run SETUP.EXE.  This will copy driver and configuration program files to the hard drive.  The registry will be setup at the same time. You must have Administrator rights to modify the registry.

To Start Driver

Once the driver is installed and the configuration program has been run for the first time the system needs to be restarted or you may start the driver from the Devices applet in the Control Panel.

Configuration Program

The configuration program ET586NT.EXE is a simple program that let's the user easily change the driver settings in the registry.  These settings are used by the driver to initialize the Evergreen 5x86 upgrade registry bits.

Words of Caution – My Edit

You must open the ET586NT program and hit OK then reboot before ET586NT appears in your Control Panel/Devices.  To be safe, set 'startup' to 'no' then reboot. Put device driver to ‘Manual’ startup before working with the driver.  Change the device start from System to 'Manual' should remove any error at boot time.  You must set “Yes, attempt to setup” before starting the device driver.

Also note that the values for the register bits in the ET586NT application are in decimal, not hex as with the DOS version of the program.  If using the Excel HEX converter, convert it into decimal before inputting the number in ET586NT.  The ET586NT program does not accept hex numbers -- the registry converts it to hex, which is then used by the driver file.

The location of the input data in the registry is:

HKEY_LOCAL_MACHINE\SYSTEM\ControlSet001\Services\ET586NT\Parameters

Using This Program

Note: Due to the complex nature of the tests conducted by this program, it may hang on some systems.  This does not denote an incompatibility with Evergreen CPU upgrades.

To select the desired option use arrow keys to highlight the option, then press the ENTER key to activate or press the hot key of the option (in red).  If a mouse is installed, it can be used to select an option by clicking once on the desired option with the left mouse button. This will highlight the option. Clicking once more will activate the option. Double-clicking can also be used to select the desired option.

Text Viewer

If you select a function that uses the text viewer, for example this Help file, it can be viewed using this program's text viewer.  Use up and down arrow keys to scroll through the text.  PAGE UP and PAGE DOWN can be used to view the text a page at a time.  Pressing the 'P' key will print the file.  Pressing the ESC key will close the window and return to the main program.  This also holds true for other popup windows used in the program.

Function Descriptions

All of the program functions give a brief description of themselves, when highlighted, on the last line of the screen.

Help

------------------

Display this help file.

Upgrade Solutions

------------------

Display possible Evergreen upgrades for computer based on test results.

Show Report

------------------

Show a report of test results and upgrade options for this computer.

Note: If you are unable to print this file while running the program, the file is saved as REPORT.TXT and can be printed at a later time using DOS PRINT or a text editor. Also, if you want to save the reports and prevent them from being over-written the next time the program runs, for example saving  test results before upgrade installation, to compare to later test results, the file REPORT.TXT will have to be renamed, because that file name is used to save results each time the program runs.

Socket Styles

------------------

Display definitions of the different CPU socket styles.

View CONFIG.SYS

------------------

Display CONFIG.SYS File.

View AUTOEXEC.BAT

------------------

Display AUTOEXEC.BAT File.

Note: The next four options display information in a window on the right side of the computer monitor.

CPU & FPU Info

------------------

Displays basic information about the computers CPU and FPU.

System Info

------------------

System Information is displayed, including information about the BIOS, type of video installed, etc.

Advanced Tests

------------------

1 - Adv. CPU ID

Gives advanced CPU identification information.

2 - CPU Config.  (NOT ACTIVE)

Displays advanced CPU register configuration information.

Note: Some advanced tests will not run on all machines and may hang system.

Ordering Info

------------------

Displays contact information to order or obtain more information about Evergreen's CPU upgrades.

Enter User Info

------------------

Enter user information (name, comments) used in Show Report.

Rerun Benchmark

------------------

Allows Dhrystones Benchmark to be rerun.

About       ALT+A

------------------

Display About screen for this program.

Exit          ESC

------------------

Exit this program. Confirmation window is displayed.

Information on Cyrix 5x86 Register Bits

Original script Written by Mr. F355 on 1998/11/15

English Translation by Sandy on 2002/09/13

5x86reg.xls is a simple Excel file used for calculating HEX values for the register parameters of ET586.EXE by Evergreen.  ET586.EXE activates registers specially defined in the Cyrix 5x86 CPU.

Excel cells which are greyed out are locked and cannot be changed. These are register bits that cannot be changed.  This program requires Microsoft Excel Ver 5.0 and higher. In order to use this program properly, please ensure that "analyzing tool" is activated in Tools – Add-In.

In the register explanation sheet, each cells of each register line indicates what type of function is assigned to the corresponding bit number on the column. Set "1" for each function you want to activate, and input "0" to deactivate. The HEX values will display automatically in the "HEX" column of the concerning register line. For instance, if you get "80" in HEX column of PCPR0 then you have to indicate the value /PCPR0=80 as a switch.

[Example 1]

Stock condition of Reply PowerBoard ( M.70 ) without cache driver.

Register/Index  	7       6       5       4       3       2       1       0       HEX

[PCR0]          	1       0       0       0       0       0       0       0       80

[CCR1]          	0       0       0       0       0       0       0       0       00

[CCR2]          	0       0       0       1       0       1       1       0       16

[CCR3]          	0       0       0       1       0       0       0       0       10

[CCR4]          	0       0       0       1       1       0       0       1       19

[SMAR0]         	0       0       0       0       0       0       0       0       00

[SMAR1]         	0       0       0       0       0       0       0       0       00

[SMAR2]         	0       0       0       0       0       0       0       0       00

[PMR]           	0       0       0       0       0       0       1       1       03

[DIR0]          	0       0       1       0       1       1       0       1       2D

[DIR1]		0       0       0       1       0       0       1       1       13

This is equivalent to the syntax below:

INSTALL=C:\ETI\ET586.EXE /PCR0=80 /CCR1=16 /CCR3=10 /CCR4=19 /PMR=3 /WBE

/WBE means Write-back Cache ENABLED (if you want to disable it, use /WBD)

Before changing switches, it is recommended that you verify the default condition of the Cyrix 5x86 in your specific motherboard. After installing ET586.exe onto your system, it is advised to:

1) Boot your system. Right after you get "starting MS-DOS" on your screen, press F5 to bypass your config.sys and autoexec.bat.

2) Change dir to where you installed Evergreen programs.  Run et586.exe with /S switch.

ET586.EXE /S

3) You will get register setting of your system as in example 1 above.

4) Take note of the HEX values for each register.

5) Reboot your system, run Windows and open 5x86reg.exl to obtain HEX values for the desired registry bits.

6) You do not need to define registers which are the same as your default system from step 3/4.

7) You may have the following line at the top of your config.sys if you installed ET586.EXE with the default installation.

INSTALL=C:\ETI\ET586.EXE /CCR4=1D

This means that the installer changed the value in CCR4 from system default "19" to "1D". The rest of the registers remain unchanged.

8) To change the settings, simply add the register values desired. For example if you want to change PCR0 to 2 and CCR2 to 22 then the top line of your config.sys should be changed as follows.

INSTALL=C:\ETI\ET586.EXE /PCR0=2 /CCR2=22 /CCR4=1D /PMR=3 /WBE

** Note: you may not include /PMR=2 and  /WBE **

The Following example is the one with which I've experienced good performance result on my  M.70 PowerBoard with bench mark programs such as DOS Dhrystones in ETDIAG.EXE by Evergreen.

[Example 2] - Best setting obtained using M.70 PowerBoard

Register/Index  	7       6       5       4       3       2       1       0       HEX

[PCR0]          	0       0       0       0       0       0       1       0       02

[CCR1]          	0       0       0       0       0       0       0       0       00

[CCR2]          	0       0       0       1       0       1       1       0       16

[CCR3]          	0       0       0       0       0       0       0       0       00

[CCR4]          	0       0       1       1       1       0       0       1       39

[SMAR0]         	0       0       0       0       0       0       0       0       00

[SMAR1]         	0       0       0       0       0       0       0       0       00

[SMAR2]         	0       0       0       0       0       0       0       0       00

[PMR]          	0       0       0       0       0       0       1       1       03

[DIR0]          	0       0       1       0       1       1       0       1       2D

[DIR1]          	0       0       0       1       0       0       1       1       13

In the above case, the following line should be included in config.sys

INSTALL=C:\ETI\ET586.EXE /PCR0=2 /CCR3=0 /CCR4=39 /PMR=3 /WBE

Additional Information

ET586.exe is not a TSR program which permanently resides in the system memory. Some cache programs are TSR types and these kinds of cache drivers will cause MS-DOS compatibility mode under Windows 95.

If you use this kind of TSR program, you may solve the problem by using the following procedure.

1)  Open  ios.ini under the windows directory.

2)  Add the drive name under the [SafeList] section, then save it.

If you set the WB function to enabled and your system frequently hangs with "Illegal Transaction" or "General Protection Error", the following procedure may help.

1) Copy WB16OFF.EXE under the Win95 dir. of your Win95 CD to the Windows dir. of your boot drive.

2) Add  "C:\WINDOWS\WB16OFF.EXE" as the last line in your autoexec.bat.

3) Reboot your system.

This text and 5x86reg.xls were written referring to the data sheet for the ST5x86, a compatible CPU with the Cyrix 5x86, which was produced by SGS-THOMSON MICROELECTORONICS.

The Register Bits

1 = Feature Enabled

0 = Feature Disabled

Default values (1 or 0) are shown in the corresponding rows and columns for a Cyrix 5x86 – 120 w/ Biostar MB-8433UUD Ver. 3.0. The bolded features were successfully enabled in Windows NT 4.0 using ET586NT.EXE (BWRT, FP_FAST, LOOP_EN, RSTK_EN).  For this configuration, the HEX and DEC values are as follows:

| Register & Index | MAPEN |  | 7 | 6 | 5 | 4 | 3 | 2 | 1 | 0 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Performance Control [PCR0] | 1h | 20h | LSSSER 1 | 0 | 0 | 0 | 0 | LOOP_EN 0 | BTB_EN 0 | RSTK_EN 0 |
| Control 1 [CCR1] | xh | C1h | 0 | 0 | 0 | 0 | MMAC 0 | SMAC 0 | USE_SMI 1 | 0 |
| Control 2 [CCR2] | xh | C2h | USE_SUSP 1 | BWRT 0 | 0 | WT1 1 | SUSP_HALT 0 | LOCK_NW 1 | USE_WBAK 1 | 0 |
| Control 3 [CCR3] | xh | C3h | MAPEN3 0 | MAPEN2 0 | MAPEN1 0 | MAPEN0 1 | SMM_MODE 1 | LINBRST 1 | NMI_EN 0 | SMI_LOCK 0 |
| Control 4 [CCR4] | 1h | E8h | 0 | 0 | FP_FAST 0 | DET_E 1 | MEM_BYP 1 | IORT2 0 | IORT1 0 | IORT0 0 |
| SMM Address [SMAR0] | xh | CDh | A31 0 | A30 0 | A29 0 | A28 0 | A27 1 | A26 1 | A25 1 | A24 0 |
| SMM Address [SMAR1] | xh | CEh | A23 0 | A22 0 | A21 0 | A20 0 | A19 1 | A18 0 | A17 1 | A16 0 |
| SMM Address [SMAR2] | xh | CRh | A15 | A14 | A13 | A12 | SIZE3 | SIZE2 | SIZE1 | SIZE0 1 |
| Power Management [PMR] | 1h | F0h | 0 | 0 | 0 | 0 | 0 | HLF_CLK 0 | CLK1 1 | CLK0 1 |
| Device ID0 [DIR0] | xh | FEh | 0 | 0 | 1 | 0 | 1 | 1 |  | 1 |
| Device ID1 [DIR1] | xh | FFh | SID3 0 | SID2 0 | SID1 0 | SID0 0 | RID3 1 | RID2 0 | RID1 1 | RID0 1 |

Register/Index  	HEX	DEC

[PCR0]		05	5

[CCR1]          	02	2

[CCR2]          	D6	214

[CCR3]          	1C	28

[CCR4]          	38	56

ET586NTonly accepts the DEC values.

Enabling BRB_EN results in an immediate BSOD.

The location of the input data in the registry is:

HKEY_LOCAL_MACHINE\SYSTEM\ControlSet001\Services\ET586NT\Parameters

Performance Control

RSTK_EN	Return Stack Enable

BTB_EN	Branch Target Buffer Enable

LOOP_EN	Loop Enable

LSSER		Load Store Serialize Enable

Configuration Control Register 1

USE_SMI	Enable SMM Pins

SMAC		System Management Memory Access

MMAC		Main Mamory Access

Configuration Control Register 2

USE_WBAK	Enable Write-Back Cache interface Pins

LOCK_NW	LOCK NW Bit

SUSP_HALT	Suspend on HALT

WT1		Write-Through Region 1  (640 Kbytes to 1 MByets)

BWRT		Enable Burst Write Cycles

USE_SUSP	Enable Suspend Pins

Configuration Control Register 3

SMI_LOCK	SMM Register Lock

NMI_EN	NMI Enable

LINBRST	Linear Address Burst Cycles

SMM_MODE	SMM MODE

MAPEEN (3 – 0)	MAP Enable

Configuration Control Register 4

IORT (2 – 0)	I/O Recovery Time

IORT2	IORT1	IORT0	I/O Recovery Time

0	0	0	no clock delay

0	0	1	2-clock delay

0	1	0	4-clock delay

0	1	1	8-clock delay

1	0	0	16-clock delay

1	0	1	32-clock delay

1	1	0	64-clock delay

1	1	1	128-clock delay

MEM_BYP	Memory read bypassing is enabled

DTE_EN	Enable Directory Table Entry Cache

FP_FAST	Fast FPU Exception Handling

Power Management Register

CLK (1 – 0)	 Core Clock/Bus Clock Ratio

CLK1	CLK0	Ratio

0	0	1/1

0	1	2/1

1	0	reserved

1	1	3/1

HLF_CLK	Half Speed Clock

Registers for suspend functions (SUSP_HALT, USE_SUSP) are supposed to be not compatible with Intel CPUs.