Program 5X86.EXE

Author Peter N Moss

E-Mail Address pmoss@yoda.alt.za

Version C2,  12 May 1996.

Source code: Source code is not included.

Compiled with Turbo Pascal V5.5 and TASM V1.1

Files:

5X86xx.ZIP      The combined zipped distribution package

5X86.EXE        The program file

CX5X86.EXE      Device driver for cache

5X86.REG        Registration form

5X86.DOC        This file

The program: 5X86.EXE

What it is not:  A magic wand.

***********************************************************************

The program is a tool that allows the user to enable the Cyrix 5X86

register bits.  It can not make any feature work, only enable it.

***********************************************************************

For use with DOS, Windows, Windows 95 and OS/2.

The program detects the presence of a 5x86 processor and will not

run if a Cyrix/IBM 5x86 is not found.

5x86.EXE will report and enable the modification of the performance

control registers of the Cyrix 5x86 (TM) processor.  Access to

register bits marked reserved (*) is not supported other than to

report status if in some cases non zero.  See Undocumented commands.

Usage: 5x86 <[/H|?] [/Q]> </BTB_EN=[on|off] /LOOP_EN=[on|off]

/LSSER=[on|off] /RSTK_EN=[on|off] /BWRT=[on|off]

/WT1=[on|off]  /LINBRST=[on|off] /MEM_BYP=[on|off]

/DTE_EN=[on|off] /FP_FAST=[on|off]> <[/ICD|/CWT|/CWB]> <[/T]>

Where BTB_EN  Branch prediction

LOOP_EN Loop (Prefetch buffer)

LSSER   Load store serialise, enable reorder

RSTK_EN Return stack

BWRT    Burst write cycle 4 dwords on cache line replacement.

WT1     Write through region one (640kb..1Mb).

LINBRST Linear burst mode.

MEM_BYP Memory read bypassing

DTE_EN  Directory table cache

FP_FAST Fast NPU exception reporting

ICD     CPU Internal cache disable

CWT     CPU cache enable, write through mode

CWB     CPU cache enable, write back mode

Q = Program version

H,? = Help

T = Status of PCR0 register

E = (Reserved) Debug report.  Status of PCR0, CCR1..4

and CR0

Note that T may be combined with register bit settings

and will report status after the register has been set

by reading the register. Thus giving confirmation that

the register has been set correctly ie 5x86 /LSSER=on

/BTB_EN=on /RTS=off /t

/E is included for debugging and expansion.  The report

is more than 25 lines so it would be advisable to use

this with either a DOS pipe to more (| more) or

redirection to a file (> filename).  It is is not part

of the program and is not expected to continue beyond

its current use.  Note CR0 NW bit cache status will

always report write through mode regardless of the

actual operating mode.  This has been changed and now

reports status 1 or 0.

On = enable, 1; Off = disable, 0.  On and off are valid

and all other values are treated as a error.

The first three characters only need be entered. LIN=on

is the same as LINBRST=on.  Less than three characters

of the register bit name will cause an error.

The cache enable/disable parameters are only valid for

the CPU at level 0.  DOS operates at level 0 (ring 0)

if EMM386 is not loaded.  No checking or testing is

done for ring 0 operation.  The report of CR0 will be

incorrect for cache status as the NW bit does not

reflect cache status.  This is a currently known fault

and will be corrected in a future version.  It is only

reported using the /e switch which is for debugging

purposes only.

Upper case or lower case characters may be used.  The

order of register input is not important.  H, ? or Q

will terminate any other command line parameters.  No

spaces can be placed between the register, equal sign

and action, ie. "BTB = on" is invalid as a space is a

valid separator.  BTB=on is the correct form.

T may be placed anywhere on the command line.

Command line input may use any legal DOS separator,

space tab / ; etc.

******************************************************************

[Undocumented commands added to version B9,C2]

/CPUID=[on|off]     on = enable the CPUID command.

/SMAC=[on|off]

/MMAC=[on|off]

/USE_SMI=[on|off]   on = enable SMM pins

/IORT=[on|off]      on = 16 clks, off = 0 clks

/AISP=[on|off]      on = all instructions stalled to serialise

pipeline

/RLML=[on|off]      on = reorder locked misalighned loads

*******************************************************************

Errorlevel:

5 = No Cyrix 5x86.

1 = Command line entry error

0 = Normal termination.

The Cyrix 5x86 CPU registers:

Full details of the register address and access requirements can be

found in the Cyrix 5x86 (TM) processor data book #94192-00.

-oo-

Register: Performance control register 0

The following bits of this register perform various functions detailed

with the register bit name.

Register PRC0 Index 20h

Access to this register is via I/O ports 22h (Index) and 23h (Data)

and the MAP_EN bits of the CCR3 register.

Bit map:

----------------------------------------------------------------

7      6      5      4      3        2        1        0

----------------------------------------------------------------

|LSSER|  Res4|  Res3|  Res2|  Res1|  LOOP_EN|  BTB_EN|  RSTK_EN|

----------------------------------------------------------------

Bit    Name     Function

7      LSSER    1 = Load store serialise enable (Reorder disable)

6      BTBT     Res4, BTB Test register

5      Res3     Reserved

4      RLML     Res2, Reordering of locked misalighned loads

3      AISP     Res1, All instructions stalled to serialise pipeline

2      LOOP_EN  Loop enable

1      BTB_EN   Branch target buffer enable

0      RSTK_EN  Return stack enable

RSTK_EN: 1 = The return stack is enabled and RET instructions will

speculatively execute the code following the associated CALL

to improve performance.

BTB_EN:  1 = Branch target buffer enable.  Branch prediction occurs.

LOOP_EN: 1 = The CPU will not flush the prefetch buffer if the

destination jump is already in the prefetch buffer

eliminating the need for a read from the cache.

LSSER:   1 = All memory reads and writes will occur in execution

order. Reordering disabled.

LSSER should be set to ensure that memory mapped I/O devices

operating outside of the address range 640k..1M will operate

correctly.

Memory accesses in the range 640k..1M will always be issued

in execution order.

-oo-

Register: Configuration control register 2

The following bits of this register perform various functions detailed

with the register bit name.

Register CCR2 Index C2h

Bit map:

----------------------------------------------------------------

7      6      5      4       3        2       1       0

----------------------------------------------------------------

|USE_SUSP| BWRT | Res2 | WT1 |SUSP_HALT|LOCK_NW|USE_WBAK| Res1 |

----------------------------------------------------------------

Bit    Name      Function

7      USE_SUSP  1 = Enable suspend pins

6      BWRT      1 = Enable use of 16 byte write back cycles

5      Res2      Reserved

4      WT1       1 = Write through region 1 (640k..1M)

3      SUSP_HALT 1 = CPU enters suspend mode after HALT instruction

2      LOCK_NW   1 = Prohibit change of state of NW bit of CR0

1      USE_WBAK  1 = Enable cache write back pins

0      Res1      Reserved

USE_WBAK: Enable INVAL and WM_RST input pins, and CACHE# and HITM#

output pins.  Must be set prior to enabling NW bit of CR0.

WT1: Forces all writes to the address region between 640k and 1M

that hit in the on-chip cache to be issued on the external bus.

BWRT: Enable use of 16 byte write back cycles.  Set only if

system logic supports.

-oo-

Register: Configuration control register 3

The following bits of this register perform various functions detailed

with the register bit name.

Register CCR3 Index C3h

Bit map:

-------------------------------------------------------------------

7       6       5       4        3        2       1       0

-------------------------------------------------------------------

|MAP_EN3|MAP_EN2|MAP_EN1|MAP_EN0 |SMM_MODE|LINBRST|NMI_EN|SMI_LOCK|

-------------------------------------------------------------------

Bit    Name     Function

7      MAP_EN3  = 0

6      MAP_EN2  = 0

5      MAP_EN1  = 0

4      MAP_EN0  1 = All configuration registers are available

3      SMM_MODE 1 = SL device compatible mode

2      LINBRST  1 = Linear address sequence is used

1      NMI_EN   1 = NMI is enabled during SMM

0      SMI_LOCK 1 = SMM register lock

SMI_LOCK: Once set can only be cleared by asserting the RESET pin.

LINBRST: Enable linear address sequence.  0 = 1 + 4 sequence, Intel

compatible.

-oo-

Register: Configuration control register 4

The following bits of this register perform various functions detailed

with the register bit name.

Register: CCR4 Index E8h

Bit map:

----------------------------------------------------------------

7       6       5       4        3       2       1       0

----------------------------------------------------------------

|CPUIDEN|  Res | FP_FAST| DTE_EN| MEM_BYP| IORT2 | IORT1 | IORT0 |

----------------------------------------------------------------

Bit    Name     Function

7      *CPUIDEN 1 = Enable CPUID instruction and EFlags bit 21

6      Res      Reserved

5      *FP_FAST 1 = Fast FPU exception reporting

4      DTE_EN   1 = Enable directory table entry cache

3      MEM_BYP  1 = Memory read bypassing is enabled

2      IORT2

1      IORT1

0      IORT0    I/O recovery time 0..128 bus clocks 2^x

* = Reserved function

CPUIDEN: Step level 1 up only.  No working sample reported.

IORT: 0 = no clock delay ... 7h = 128 clock delay.

-oo-

Usage and optimisation:

In setting the CPU please note the the results are very dependent on

the correct setup of the CMOS parameters.  Particular attention should

be placed on the L1, L2 cache settings as well as memory read and

write wait states.  It is suggested that these setting be optimised

first before attempting to "tune" the CPU performance control register.

There maybe some interaction of the CMOS settings and CPU settings

requiring iteration to find the most suitable.

The Cyrix 5x86 as other Cyrix processors requires that the motherboard

and BIOS support the on chip cache (L1) as well as other signal pins.

For this reason the motherboard jumpers and CMOS must be set correctly.

Before starting it is a good idea to run 5x86 /e to see what register

bits have been set by the BIOS.

------------------------------------------------------------------

Internal cache enable option

------------------------------------------------------------------

The cache is enabled/disabled by setting CPU CR0 register which can

only be accessed (written) when the CPU is operating at CPL 0 or ring

0.  This feature is not to be used in protected mode (32 bit programs)

such as Windows, Unix etc.  No checking is done at present although

possible.  The user is expected to read and understand this statement

and not foolishly attempt to modify the cache setting when operating

the CPU at any other level than 0.  The use of this feature is only

included for those motherboards without Cyrix cache support as a test

at present.  Note running EMM386.EXE and similar memory managers places

DOS at CPL 3 and you will not be able to change the setting of CR0.  A

clean DOS boot is absolutely essential to use ICD, CWT and CWB.

CBW will enable the write back pins of the CPU and if these are not

supported by the hardware, writeback mode can not be expected to work.

CX5X86.EXE is included so that if it is required that the internal

cache be manually enabled, this can be done in CONFIG.SYS.  This

device driver has no other purpose.

See device driver, cx5x86.exe

------------------------------------------------------------------

Some problems have been noted with LSSER set to 0 the default when

other functions (bits) have been enabled.

Suggested BIOS default values are:

CCR2  BWRT    = 0 System logic must support

CCR4  MEM_BYP = 1

DTE_EN  = 1

FP_FAST = 0

IORT    = 0

PCR0  RSTACK  = 0

BTB     = 0

LOOP    = 0

LSSER   = 1 for PCI memory mapped devices

LSSER   = 0 for systems without PCI

These are a very good starting point.

The parameters for use can be found by entering h or ? on the command

line.  When the optimum parameters have been found path\5x86 {parameters}

may be placed in your autoexec.bat file with those parameters so that

the PC is set on startup.

As a guide for current revisions of 5x86 CPU enable the following

registers if not enabled by the BIOS.  Note BTB may not work for all

operating systems and hardware.

BTB_EN=on   may cause problems with windows....

MEM_BYP=on  safe setting

DTE_EN=on   safe setting

FP_FAST=on  safe setting

WT1=on      if writeback L1 is selected in CMOS

USE_WBAK=on writeback mode only

BWRT=on     writeback mode only

LSSER=on    for PCI or if instability is noted.

Registers that should be enabled by the BIOS

Write through mode:

MEM_BYP

DTE_EN

LSSER for PCI, see above

Write back mode:

USE_WBAK

BWRT      Some MBs will not operate with this setting

WT1

MEM_BYP

DTE_EN

LSSER for PCI, see above

It may be found that if the BIOS is Cyrix 5x86 aware that some of these

registers are enabled.  Notably MEM_BYP and DTE_EN, with USE_WBAK, WT1

and sometimes BWRT enabled if the cache is in writeback mode and burst

writes have been enabled.  If BWRT is enabled disable burst write in

the CMOS and check again.  It is a possible area for failure to

operate.  The only way to find out is to check with 5x86 /e and note

any enabled registers.  These settings were tested with IBM5x86-100 step

1, rev 3 and an Asustek SV2GX4-VL motherboard.  BWRT or burst writes

would not operate on this motherboard and required that CMOS setting

for burst writes be disabled.

OS/2:

In startup.cmd

start /c /dos /win [drive:\path\5x86 <parameters>]

@exit

This is untested and due to my total ignorance of OS/2 may contain errors

Your manuals and documentation will be of use here, I will not.

-oo-

Optimisation:

Part of my problem in writing documentation is the level to aim it at.

I am sure that anyone can appreciate that it is really not possible

to bring up to speed all users of the program.  Microsoft even fails

at that.  The setting of the advanced features does require an

understanding of at least some of the technicalities and processes.

As does setting the CMOS.  Use benchmarks such as Landmark SPEED V2,

Norton SI and Ziff-Davis PCBENCH V9 for your testing as these have

published results.

Step 1:

Start by setting the CMOS/BIOS such that maximum performance is

obtained.  This is to check that the system is functional to a proper

level.  The results obtained should be similar to any 486DX4 at the

same speed.  If this can not be obtained there is no purpose in

feeding the system purple hearts by setting the advanced features in

the hopes of making a sick child well.  Note what if any features the

BIOS enables.  Step 2 may not be required.

Step 2:

Once all problems have been resolved, enable MEM_BYP and DTE_EN.

If the system is a PCI bus enable LSSER.  This is the basis of

advanced features and the setting used by most BIOSs that support the

Cx5x86.  Performance should be at or close to the Cyrix published

figures.  If not go back to step 1.

Step 3:

Enable BTB and test at this point you may have to go back to step 1

as some settings may have been on the edge and will now fail.  Pay

attention to cache and dram settings.  There are many owners who can

not operate with branch prediction enabled, especially in windows or

windows 95.  The revision of your CPU may be a factor here and step 1

revision 3 up should be suitable but is not guaranteed.

Step 4:

Disable LSSER if it was enabled.  Enable WT1.  Some chipsets

support linear burst mode and enabling this may be possible.  These

are listed.

ALI          M1489/87

OPTI         82C465MVB

Picopower    868/818

668/618

768/718

Symphony     491/492 Wagner

UMC          8880

Anyone having success with linear burst mode please report it fully.

Step 5:

You are on your own.  Note that BTB is not guaranteed to work nor

are steps 3 and 4 and there are many MBs+hardware combinations that

will not run correctly.  There is no magic formulae to sort yours out.

It takes hard work and much testing.  Keep notes of what has been done

and the results.  Even without BTB enabled this is one of the fastest

486 class processors available.  It may be possible to run BTB enabled

in DOS but not in Windows.

Notes:

I have assumed that the mother board supports writeback L1 cache.

If you don't understand a particular aspect ask.  A general query such

as "my 5x86 will not work, what is wrong?" will be ignored as nobody

has that much time.  Give as much detail as will be required to answer

the question.

-oo-

What are we dealing with?

Memory management:

Load-store reordering; that prioritises memory reads required by the

integer unit over writes to external memory.

Memory-read bypassing; that eliminates unnecessary memory reads by using

valid data still in the execution unit.

Branch prediction:

The branch prediction target buffer (BTB) is used to store branch target

addresses and branch prediction information.

The target address of a RET instruction is dynamic rather than static

and return addresses are cached in a return stack rather than the BTB.

The return address is pushed on the return stack during a CALL

instruction and popped during the corresponding RET instruction.

-oo-

The device driver : CX5X86.EXE

USE: Only if the BIOS does not enable writeback mode and the system

logic supports write back mode.

Use in config.sys file as the FIRST device loaded.  It is of no use at

all if loaded after EMM386 and similar programs.  It is intended for

older motherboards that do not enable the cache.  Write back mode may

not be usable.  See Usage and optimisation section.

Usage: DEVICE=[Path\]CX5X86.EXE <[/CD|/WT|/WB]>

Where /CD = Cache disable

/WT = Cache enable in write through mode

/WB = Cache enable in write back mode

The syntax is exact; "/" must be used to proceed the option

and only the options as given are valid.  Upper or lower case

characters may be used.  Only one option (the first) on the

config.sys "device = CX5x86" line is valid.

EXAMPLE:Device=c:\util\cx5x86.exe /wt

(Write through mode selected)

Write through mode sets the CR0 register and LOCK_NW bit of CCR2.

Write back mode sets the CR0 register and USE_WBAK and LOCK_NW bits

of CCR2.  This will cause problems if the hardware does not support

the write back configuration.  Your motherboard supplier or

manufacture should be able to answer this question.  If not are you

buying from the right company?

Cache disable sets CR0 bit CD.

This device driver is not installed and thus takes up no memory

after use.  It can only be used with DOS, Windows and Windows 95.

-oo-

References:

Cyrix 5x86(TM) Microprocessor Data Book

IBM 5x86 BIOS Writers Guide

CTCHIPZ configuration files

Functionality, type and rights:

5x86.EXE is not crippled or contain nagware and relies completely on

the honesty of the user.  Use of the program is not free and the user

is both legally and morally bound to registration and payment of the

registration fee.

5x86xx.ZIP is shareware, all rights are reserved by Peter N Moss.

Copyright:

5x86.EXE, Cx5x86.EXE and 5x86.DOC are copyright 1995 Peter N Moss.

Disclaimer:

No claim is made for the fitness of purpose of this software and no

claim for damages of any nature will be accepted.  The program is

for use at entirely your own risk.

Distribution:

You may distribute the files only on condition that they are not

altered and all files are included.  No charge may be made for the

software.  The latest version can be found at

ftp://compnt.cs.unp.ac.za/uploads/cyrix/5x86(Ver).zip.

And at http://www.dfw.net/~sdw/cxstuff.html  This site may lag the

ftp site.

Updates to registered users will only be sent via Internet e-mail

without charge.  Disk updates will be charged at the cost of

materials and postage.

Reports and support:

Whilst reports of results or bugs are welcomed, it is not possible to

support questions on how to setup or optimise any motherboards.  You

are referred to the dealer that sold the board and or CPU.  Registered

users can expect free update notification and as much help as can be

given.  Unregistered users can expect the same consideration as they

have given the author.

Thanks:  Mr C Chan, Mr K J Wren, Mr J Stiel and Mr C D Regenberg for

testing and feedback.  Cyrix Corp for data.  IBM for data.

Dave Harter of Cyrix for help and information.  Paul Naylor

of Cyrix for data.

Trademarks:  Cyrix is a registered trademark of Cyrix Corp.

Cx5x86 is a registered trademark of Cyrix Corp.

IBM is a registered trademark of International Business

Machines.

Windows and windows 95 are registered trademarks of

Microsoft Corp.

Payment:

You are offered 30 days free trial for evaluation only.  After that

period has expired and registration or payment has not taken place

then the program must not be used.

Personal/corporate cheques are the preferred method of payment.

Payment of US$15.00 or the equivalent is such a small amount that

anyone that uses the program continuously should contribute to the

development of it.  Approximately 500 man hours have gone into the

development of the programs.  It will also insure that the program

continues to be developed.

Your honesty will ensure the continued development of shareware

programs, in particular 5x86.  No shareware author wishes to donate

his time and effort to people unwilling to appreciate that effort.

Would you offer a similar amount of time or effort to the shareware

author?  If not, please pay the small amount requested.  Not doing so

is theft.

For money order payment from outside of South Africa please add $4.00.

Payment in cash, cheques (preferred) or money order may be made to:

P N Moss

P O Box 212144

Oribi

Pietermaritzburg

Rep of South Africa, 3205

When making payment please include your e-mail address if you have one

and your return mail address.

Registration form:  5X86.REG